ADVANTEST PA Spectrum Analyzers (VISA)

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for ADVANTEST PA Spectrum Analyzers (VISA).

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

ADVANTEST PA Spectrum Analyzers (VISA)

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

ADVANTEST PA Spectrum Analyzers (VISA)  Function Name:

Initialize                                  AVPA_init

Application Functions

Configure and Read                       AVPA_configRead

Configuration Functions

Configure Frequency

Set Frequency                         AVPA_Set_Freq

Set Frequency Span                    AVPA_Set_Span

Set Frequency Offset                  AVPA_Set_FreqOffset

Set Frequency Step Size               AVPA_Set_FreqStepSize

Set Frequency Counter                 AVPA_Set_FreqCounter

Configure Amplitude

Set Reference Level                   AVPA_Set_RefLevel

Set Reference Unit                    AVPA_Set_RefUnit

Set Scale                             AVPA_Set_Scale

Set Reference Level Offset            AVPA_Set_RefOffset

Set Input Attenuation                 AVPA_Set_Att

Set Minimum Input Attenuation         AVPA_Set_MinATT

Configure Bandwidths

Set RBW                               AVPA_Set_RBW

Set VBW                               AVPA_Set_VBW

Set RBW/Span Coupling                 AVPA_Set_RBW_Span

Set RBW/VBW Coupling                  AVPA_Set_RBW_VBW

Set All Auto Couple                      AVPA_Set_All_Auto_Couple

Configure Sweep

Set Sweep Time                        AVPA_Set_SweepTime

Set Sweep Mode                        AVPA_Set_SweepMode

Set Trigger Mode                      AVPA_Set_TriggerMode

Set Gated Sweep                       AVPA_Set_GatedSweep

Configure Trace

Set Trace A Mode                      AVPA_Set_TraceAMode

Set Trace B Mode                      AVPA_Set_TraceBMode

Set Trace Detector Mode               AVPA_Set_TraceDetector

Set Trace Points                      AVPA_Set_TracePoints

Configure Display & Limit Lines

Set Limit Line                        AVPA_Set_LimitLine

Activate Limit Line                   AVPA_Activate_LimitLine

Move Limit Line                       AVPA_Move_LimitLine

Set Display Line                      AVPA_Set_DisplayLine

Configure Measurement Window

Set Measurement Window                AVPA_Set_MeasurementWindow

Configure Level Correction

Set Level Correction                  AVPA_Set_LevelCorrection

Configure Markers Normal&Delta

Set Marker Normal                     AVPA_Set_MarkerNormal

Set Marker Delta                      AVPA_Set_MarkerDelta

Set Marker Delta Options              AVPA_Set_MarkerDeltaOptions

Set Marker Trace                      AVPA_Set_MarkerTrace

Configure Multimarkers

Activate Multimarkers                 AVPA_Activate_Multimarkers

Set Multimarkers                      AVPA_Set_Multimarkers

Set Active Multi Marker               AVPA_Set_ActiveMultiMarker

Delete All Markers                    AVPA_Delete_Markers

Configure Peak Search

Set Peak Search Options               AVPA_Set_PeakSearchOptions

Action/Status Functions

Execute Peak Search                      AVPA_Search_Peak

Execute Peak To                          AVPA_Execute_PeakTo

Marker Delta To Settings                 AVPA_Set_MarkerDeltaTo

Trace Action

Take a Sweep                          AVPA_Take_Sweep

Execute Trace Math                    AVPA_Execute_TraceMath

Start/Stop Trace Average              AVPA_Control_TraceAverage

Configuration Read

Read Frequency Settings               AVPA_Read_FreqSettings

Read Frequency Offset                 AVPA_Read_FrequencyOffset

Read Frequency Step Size              AVPA_Read_FreqStepSize

Read Frequency Counter                AVPA_Read_FreqCounter

Read Reference Level                  AVPA_Read_ReferenceLevel

Read Reference Level Offset           AVPA_Read_RefLevelOffset

Read Reference Unit                   AVPA_Read_RefUnit

Read Scale                            AVPA_Read_Scale

Read Attenuation                      AVPA_Read_Attenuation

Read RBW&VBW                          AVPA_Read_RBWandVBW

Read Sweep Time                       AVPA_Read_SweepTime

Read Sweep Mode                       AVPA_Read_SweepMode

Read Trigger Mode                     AVPA_Read_TriggerMode

Read Gated Sweep                      AVPA_Read_GatedSweep

Read Display Line                     AVPA_Read_DisplayLine

Read Limit Line Options               AVPA_Read_LimitLineOptions

Read Measurement Window               AVPA_Read_MeasurementWindow

Read Peak Options                     AVPA_Read_Peak_Options

Status Read

Read Trace Status                     AVPA_Read_TraceStatus

Read Detection Mode                   AVPA_Read_DetectionMode

Read Trace Points                     AVPA_Read_TracePoints

Read Limit Line Status                AVPA_Read_LimitLineStatus

Read Limit Line Judgment              AVPA_Read_LLjudgment

Read Level Correction Status          AVPA_Read_LevelCorrectionStatus

Read Marker Status                    AVPA_Read_MarkerStatus

Read Marker Delta Options             AVPA_Read_MarkerDeltaOptions

Read Marker Trace Status              AVPA_Read_MarkerTraceStatus

Read Multimarkers Status              AVPA_Read_MultimarkersStatus

Data Functions

Trace Function

Read Trace Data                       AVPA_Read_TraceData

Markers Functions

Read Marker Normal                    AVPA_Read_MarkerNormal

Read Marker Delta                     AVPA_Read_MarkerDelta

Read Multimarkers                     AVPA_Read_Multimarkers

Read Multi Marker Active              AVPA_Read_MultiMarkerActive

Utility Functions

Write To Instrument                      AVPA_WriteInstrData

Read Instrument Data                     AVPA_readInstrData

Reset                                    AVPA_reset

Self-Test                                AVPA_selfTest

Error-Query                              AVPA_errorQuery

Error Message                            AVPA_errorMessage

Revision Query                           AVPA_revisionQuery

Calibration                              AVPA_Calibration

Display Label                            AVPA_Display_Label

Copy Screen                              AVPA_Copy_Screen

Read Status Byte                         AVPA_Read_StatusByte

Clear Status Byte                        AVPA_Clear_StatusByte

Frequency or Time Conversion

Frequency Conversion                  AVPA_Convert_Frequency

Time Conversion                       AVPA_Convert_Time

Memory Card Management

Initialize Memory Card                AVPA_Init_MemoryCard

Select Memory Drive                   AVPA_Select_MemoryDrive

Copy Memory Card                      AVPA_Copy_MemoryCard

Recall File                           AVPA_Recall_File

Save File                             AVPA_Save_File

Delete File                           AVPA_Delete_File

Close                                       AVPA_close

________________________________________________________________________________

ADVANTEST PA Spectrum Analyzers (VISA)

This instrument module provides programming support for the Advantest PA

Series Spectrum Analyzers R3263, R3272, R3465, and R3463. The module is

divided into the following functions:

Functions/Classes:

(1) Initialize:

This function initializes the instrument and sets it to a default

configuration.

(2) Application Functions: (Class)

This class contains high-level, test and measurement routines.  These

examples call other instrument driver functions to configure, start, and

read from the instrument.

(3) Configuration Functions: (Class)

This class of functions configures the instrument by setting acquisition

and system configuration parameters.

(4) Action/Status Functions: (Class)

This class of functions begins or terminates an acquisition. It also

provides functions which allow the user to determine the current status

of the instrument.

(5) Data Functions: (Class)

This class of functions transfers data to or from the instrument.

(6) Utility Functions: (Class)

This class of functions provides lower level functions to communicate

with the instrument, and change instrument parameters.

(7) Close:

This function takes the instrument offline.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

AVPA_Activate_LimitLine

ViStatus AVPA_Activate_LimitLine (ViSession instrumentHandle,

ViBoolean limitLine1Status,

ViBoolean limitLine2Status,

ViBoolean levelXaxis, ViBoolean levelYaxis);

Purpose

This function is used to configure the general control parameters of both

limit lines.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

limitLine1Status

Variable Type       ViBoolean

This binary switch is used to activate or desactivate the limit line

1.

Valid Range:

VI_ON - Active

VI_OFF - Inactive (Default)

limitLine2Status

Variable Type       ViBoolean

This binary switch is used to activate or desactivate the limit line

2.

Valid Range:

VI_ON - Active

VI_OFF - Inactive (Default)

levelXaxis

Variable Type       ViBoolean

Chooses between absolute and relative limit line definition with

respect with the X axis (Time or Frequency).

Valid Range:

VI_OFF - Absolute (Default)

VI_ON - Relative

Note : Relative data are based on "Delay time" when dealy sweep is

ON.

levelYaxis

Variable Type       ViBoolean

Chooses between absolute and relative limit line definition with

respect with the Y axis.

Valid Range:

VI_OFF - Absolute (Default)

VI_ON - Relative

Note : Relative data are based whether on the reference level if

display line is inactive whether on display line level if it is

active.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Limit Line 1 Status) out of range.

BFFC0003  Parameter 3 (Limit Line 2 Status) out of range.

BFFC0004  Parameter 4 (Level/X-axis) out of range.

BFFC0005  Parameter 5 (Level/Y-axis) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Activate_Multimarkers

ViStatus AVPA_Activate_Multimarkers (ViSession instrumentHandle);

Purpose

This function is used to activate the Multimarkers. They are placed at

their last known positions

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Calibration

ViStatus AVPA_Calibration (ViSession instrumentHandle,

ViInt16 elementtoCalibrate,

ViReal64 calibrationLevel,

ViBoolean calibrationCorrection,

ViBoolean frequencyCorrection);

Purpose

Specifies the type of calibration required and starts it. Don't forget to

link the output connector CAL OUT and the input connector

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

elementtoCalibrate

Variable Type       ViInt16

specifies which element to calibrate

Valid Range :

0 - All (Default)

1 - Total Gain

2 - Input Attenuator

3 - IF step Amplifier

4 - RBW switch

5 - Logarithmic linearity

6 - Amplifier Magnitude

7 - PBW

calibrationLevel

Variable Type       ViReal64

specifies the level of the output signal of calibration.

Valid Range: -10 dB to -30 dB (Default)

calibrationCorrection

Variable Type       ViBoolean

specifies whether to activate or desactivate the calibration factor

obtain after the calibration.

Valid Range:

VI_ON - ON

VI_OFF - OFF

frequencyCorrection

Variable Type       ViBoolean

specifies whether to activate or desactivate the frequency

correction.

Valid Range:

VI_ON - ON (Default)

VI_OFF - OFF

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Element to Calibrate) out of range.

BFFC0003  Parameter 3 (Calibration Level) out of range.

BFFC0004  Parameter 4 (Calibration Correction) out of range.

BFFC0005  Parameter 5 (Frequency Correction) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Clear_StatusByte

ViStatus AVPA_Clear_StatusByte (ViSession instrumentHandle);

Purpose

This function is used to clear the status byte register.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_close

ViStatus AVPA_close (ViSession instrumentHandle);

Purpose

This function performs the following operations:

viClose (instrSession) and viClose (rmSession).

Note : the instrument must be reinitialized to use it again.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_configRead

ViStatus AVPA_configRead (ViSession instrumentHandle, ViBoolean tracetoRead,

ViInt16 typeofSearch, ViInt16 traceDataArray[],

ViReal64 *sweepTime, ViChar sweepUnit[],

ViReal64 *markerPosition, ViReal64 *markerLevel);

Purpose

This function performs the following operations:

1 - returns the trace Data of the chosen trace in an array.

2 - reads the sweep time : value and unit.

3 - searches the desired peak and returns its coordinates.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

tracetoRead

Variable Type       ViBoolean

selects the trace to store in an array.

Valid Range :

VI_ON - Trace A (Default)

VI_OFF - Trace B

typeofSearch

Variable Type       ViInt16

selects the peak marker search desired

Valid Range :

0 - peak search (Default)

1 - next peak

2 - next peak left

3 - next peak right

4 - min search

5 - next minimum

traceDataArray

Variable Type       ViInt16[]

This is the array where the function will write the trace data.

sweepTime

Variable Type       ViReal64 (passed by reference)

returns the value of the sweep time value.

sweepUnit

Variable Type       ViChar[]

returns the sweep time unit.

markerPosition

Variable Type       ViReal64 (passed by reference)

returns the position of the marker normal ie the position of the

peak searched.

markerLevel

Variable Type       ViReal64 (passed by reference)

returns the level of the marker normal ie the level of the peak

searched.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace to Read) out of range.

BFFC0003  Parameter 3 (Type of Search) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Control_TraceAverage

ViStatus AVPA_Control_TraceAverage (ViSession instrumentHandle,

ViInt16 traceAverage);

Purpose

This function is used to control start, stop, pause or continue of trace

average (A).

In function "Set Trace A Mode", if Trace Average mode is selected,

averaging will start when the function is executed.

Here Trace Average can be stopped, and restarted at will without

redefining the trace count or execution mode.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

traceAverage

Variable Type       ViInt16

Specifies the action to perform in respect to Trace Averaging (A).

Valid Range:

0 - Start (Default)

1 - Stop

2 - Pause

3 - Continue

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace Average) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Convert_Frequency

ViStatus AVPA_Convert_Frequency (ViSession instrumentHandle,

ViReal64 inputFrequencyValue,

ViReal64 *outputFrequencyValue,

ViChar frequencyUnit[]);

Purpose

This function is used to convert the frequency value returned by the

spectrum analyzer X Hz to X Hz or Y kHz/MHz/GHz.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

inputFrequencyValue

Variable Type       ViReal64

Frequency value returns by the spectrum analyzer and always expressed

in Hz.

outputFrequencyValue

Variable Type       ViReal64 (passed by reference)

returns X the value of the frequency in the following expressions : X

Hz, X kHz, X MHz and X GHz.

frequencyUnit

Variable Type       ViChar[]

returns the best frequency's unit between Hz, kHz, MHz and GHz.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Convert_Time

ViStatus AVPA_Convert_Time (ViSession instrumentHandle, ViReal64 inputTimeValue,

ViReal64 *outputTimeValue, ViChar timeUnit[]);

Purpose

This function is used to convert the time value returned by the spectrum

analyzer X s to X s or Y ms,Y us, Y ns.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

inputTimeValue

Variable Type       ViReal64

time value returns by the spectrum analyzer and always expressed in

seconds.

outputTimeValue

Variable Type       ViReal64 (passed by reference)

returns X the value of the time in the following expressions : X s, X

ms, X us or X ns.

timeUnit

Variable Type       ViChar[]

returns the best time's unit between s, ms, us, and ns.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Copy_MemoryCard

ViStatus AVPA_Copy_MemoryCard (ViSession instrumentHandle);

Purpose

This fonction is used to copy memory card A to memory card B. All the

memory card is copied at once.

Note : no parameter are needed.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Copy_Screen

ViStatus AVPA_Copy_Screen (ViSession instrumentHandle, ViInt16 copyMode);

Purpose

This function is used to start a screen copy toward the printer. The

aspect of the screen copy is selected at the same time.This function

allows to start a screen copy toward the printer. The aspect of the

screen copy is selected at the same time.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

copyMode

Variable Type       ViInt16

Selects the type of print output desired.

Valid Range:

0 - Gray Scale (A4 size)

1 - Large Black & White (A4 size)

2 - Small Black & White (screen reduction)

Default Value:

1 - Large Black & White

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Copy Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Delete_File

ViStatus AVPA_Delete_File (ViSession instrumentHandle, ViBoolean deleteType,

ViInt16 registerName, ViString fileName);

Purpose

This function is used to delete a file or a register.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

deleteType

Variable Type       ViBoolean

This binary switch is used to choose whether you want delete a

register or a file.

Valid Range :

VI_ON - File

VI_OFF - Register (Default)

registerName

Variable Type       ViInt16

selects the register to delete.

Valid Range :

1 - REG_01 (Default)

2 - REG_02

3 - REG_03

4 - REG_04

5 - REG_05

6 - REG_06

7 - REG_07

8 - REG_08

9 - REG_09

10 - REG_10

fileName

Variable Type       ViString

Contains the name (8 characters maxi) of the file to delete.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Delete Type) out of range.

BFFC0003  Parameter 3 (Register Name) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Delete_Markers

ViStatus AVPA_Delete_Markers (ViSession instrumentHandle);

Purpose

This function is used to desactivate all the markers that are active.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Display_Label

ViStatus AVPA_Display_Label (ViSession instrumentHandle, ViBoolean showLabel,

ViString label);

Purpose

This function is used to display a label on the screen or to delete it.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

showLabel

Variable Type       ViBoolean

This binary switch is used to display the specified label or to

delete the current label.

Valid Range:

VI_ON - Show

VI_OFF - Delete (Default)

Note : If the current label is deleted, the label is lost and a new

label will have to be specified the next time a label must be shown.

label

Variable Type       ViString

Specifies the label to display on the upper left of the screen.

Data Range : A character string of at most 30 characters.

Note : This string is ignored if "Show Label" is set to "delete".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Show Label) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_errorMessage

ViStatus AVPA_errorMessage (ViSession instrumentHandle, ViStatus statusCode,

ViChar message[]);

Purpose

This function takes the Status Code returned by the instrument driver

functions, interprets it and returns it as a user readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  VI_NULL

statusCode

Variable Type       ViStatus

This control accepts the Status Code returned from the instrument

driver functions.

Default Value:

0 - VI_SUCCESS

message

Variable Type       ViChar[]

This control returns the interpreted Status Code as a user readable

message string.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0085  VI_WARN_UNKNOWN_STATUS

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_errorQuery

ViStatus AVPA_errorQuery (ViSession instrumentHandle, ViInt32 *errorCode,

ViChar errorMessage[]);

Purpose

This function reads an error code from the instrument's error queue.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

This control returns the error code read from the instrument's error

queue.

errorMessage

Variable Type       ViChar[]

This control returns the error message string read from the

instrument's error message queue.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Execute_PeakTo

ViStatus AVPA_Execute_PeakTo (ViSession instrumentHandle, ViBoolean peakTo);

Purpose

This function is used to assign the peak value to one of the following

parameters : Center Frequency or Reference Level.

If Peak To -> Center Frequency, the frequency of the peak becomes the

center frequency.

If Peak To -> Reference Level, the level of the peak becomes the

reference level.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

peakTo

Variable Type       ViBoolean

selects the element which will be modified with the peak value.

Data range :

VI_ON - Center Frequency (Default)

VI_OFF - Reference Level

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Peak To) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Execute_TraceMath

ViStatus AVPA_Execute_TraceMath (ViSession instrumentHandle, ViInt16 traceMath);

Purpose

This function allows to work with the traces A and B.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

traceMath

Variable Type       ViInt16

Specifies the kind of trace math function that should be activated.

Valid Range:

0 - None

1 - Exchange A & B

2 - A = A - DL  (DL = Display Line)

3 - A = A - B

4 - A = B - A

Default Value:

0 - None

Note : The selection "None" means that trace math is cancelled.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace Math) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_init

ViStatus AVPA_init (ViRsrc resourceName, ViBoolean IDQuery,

ViBoolean resetDevice, ViSession *instrumentHandle);

Purpose

This function performs the following initialization actions:

- Opens a session to the Default Resource Manager resource and a session

to the specified device using the interface and address specified in the

Resource_Name control.

- Performs an identification query on the Instrument.

- Resets the instrument to a known state.

- Sends initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

- Returns an Instrument Handle which is used to differentiate between

different sessions of this instrument driver.

- Each time this function is invoked a Unique Session is opened.  It is

possible to have more than one session open for the same resource.

Parameter List

resourceName

Variable Type       ViRsrc

This control specifies the interface and address of the device that

is to be initialized (Instrument Descriptor). The exact grammar to be

used in this control is shown in the note below.

Default Value:  "GPIB::1"

Notes:

(1) Based on the Instrument Descriptor, this operation establishes a

communication session with a device.  The grammar for the Instrument

Descriptor is shown below.  Optional parameters are shown in square

brackets ([]).

(2) Interface   Grammar

------------------------------------------------------

GPIB        GPIB[board]::primary address[::secondary address]

[::INSTR]

ASRL        ASRL[board][::INSTR]

The GPIB and ASRL keywords are used respectively with GPIB and serial

instruments.

The default value for optional parameters for GPIB are shown below.

Optional Parameter          Default Value

-----------------------------------------

board                       0

secondary address           none - 31

(3) the serial interface is initialized with 8 databits, 1 stopbit,

no parity, baud rate = 19200, flow control = XonXoff and timeout =

20000 ms

IDQuery

Variable Type       ViBoolean

This control specifies if an ID Query is sent to the instrument

during the initialization procedure.

Valid Range:

VI_OFF (0) - Skip Query

VI_ON  (1) - Do Query (Default Value)

Notes:

(1) Under normal circumstances the ID Query ensures that the

instrument initialized is the type supported by this driver. However

circumstances may arise where it is undesirable to send an ID Query

to the instrument.  In those cases; set this control to "Skip Query"

and this function will initialize the selected interface, without

doing an ID Query.

resetDevice

Variable Type       ViBoolean

This control specifies if the instrument is to be reset to its

power-on settings during the initialization procedure.

Valid Range:

VI_OFF (0) - Don't Reset

VI_ON  (1) - Reset Device (Default Value)

Note : If you do not want the instrument reset. Set this control to

"Don't Reset" while initializing the instrument.

instrumentHandle

Variable Type       ViSession (passed by reference)

This control returns an Instrument Handle that is used in all

subsequent function calls to differentiate between different sessions

of this instrument driver.

Notes:

(1) Each time this function is invoked a Unique Session is opened.

It is possible to have more than one session open for the same

resource.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (ID Query) out of range.

BFFC0003  Parameter 3 (Reset Device) out of range.

BFFC0011  ID Query Failed VI_ERROR_FAIL_ID_QUERY

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Init_MemoryCard

ViStatus AVPA_Init_MemoryCard (ViSession instrumentHandle,

ViBoolean memoryCardDrive);

Purpose

This fonction is used to initialize a memory card in either drive A or B

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

memoryCardDrive

Variable Type       ViBoolean

specifies the default memory card drive.

Valid Range :

VI_ON - Drive A (Default)

VI_OFF - Drive B

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Memory Card Drive) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Move_LimitLine

ViStatus AVPA_Move_LimitLine (ViSession instrumentHandle, ViBoolean shiftType,

ViReal64 shiftValue);

Purpose

This function is used to shift both limit lines either along the X_axis

(frequency or time shift) or along the Y_axis (level) shift.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

shiftType

Variable Type       ViBoolean

Selects the type of shift needed.

Valid Range:

VI_OFF - Level (Default)

VI_ON -  Frequency / Time shift

Notes :

(1) shift along the X axis = frequency or time shift

(2) shift along the Y axis = level shift

shiftValue

Variable Type       ViReal64

Specifies the size of the shift that should be made.

Valid Range:No

Default Value: 0 with the good unit

Note : Unit depends on the shift type. "Hz" or "sec" for the X axis

shifts, and the unit of the reference level for the Y axis shifts.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Shift Type) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_readInstrData

ViStatus AVPA_readInstrData (ViSession instrumentHandle,

ViInt32 numberBytesToRead, ViChar readBuffer[],

ViInt32 *numBytesRead);

Purpose

This function reads data from the instrument's output buffer and returns

it to the specified variable in memory.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) If valid data is not available at the instrument's output buffer when

this function is called the instrument will hang up and the function will

not return until it times out.  If the time-out is disabled this function

will hang indefinitely and it may be necessary to reboot the computer to

break out.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberBytesToRead

Variable Type       ViInt32

The number of bytes specified by this control should be greater than

or equal to the number of bytes which are to be read from the

instrument. If the actual number of bytes to be read is greater than

the number this control specifies then multiple reads will be

required to empty the instrument's output buffer.

If the instrument's output buffer is not emptied the instrument may

return invalid data when future reads are performed.

Default Value:  50 (See NOTE 2)

Notes:

(1) If NO DATA is available at the instrument's output buffer when

this function is called the instrument will hang up and the function

will not return until it times out.  If the time-out is disabled,

this function will hang indefinitely and it may be necessary to

reboot the computer.

(2) If the number of bytes expected is greater than 50 the value of

this control may be increased. If the actual number of bytes read is

larger than the declared size of the read buffer a run-time error

will be generated.

readBuffer

Variable Type       ViChar[]

The incoming data from the instrument is placed into this variable.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) This function does not overwrite any old data left in the string

variable from the last time the function was called. String data in

LabWindows/CVI is terminated with an ASCII null(0x0) and string

manipulation functions will only recognize data before the ASCII

null.

(3) The declared size of the string variable must be greater than the

actual number of bytes read from the instrument if it is not a

run-time error will be generated.

numBytesRead

Variable Type       ViInt32 (passed by reference)

This variable contains the actual number of bytes read from the

instrument. This is the value which is returned by the read function.

Notes:

(1) If the actual number of bytes read is less than the number of

bytes specified in the Number Bytes To Read control then the output

buffer has probably been emptied. If the read function fails and the

number of bytes read is 0, the most probable cause for the failure is

there was no data available at the instrument's output buffer.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_Attenuation

ViStatus AVPA_Read_Attenuation (ViSession instrumentHandle,

ViBoolean attenuationtoRead,

ViReal64 *valueindB, ViInt16 *typeofControl);

Purpose

This function reads the value of the input attenuation and the minimum

input attenuation and returns whether they are controled by the user or

by the spectrum analyzer.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

attenuationtoRead

Variable Type       ViBoolean

selects the type of attenuation to check

Valid Range :

VI_ON - Input Attenuation (Default)

VI_OFF - Minimum Input Attenuation

valueindB

Variable Type       ViReal64 (passed by reference)

returns the value always expressed in dB of the Input Attenuation or

of the Minimum Input Attenuation.

typeofControl

Variable Type       ViInt16 (passed by reference)

returns whether the Input Attenuation is set up by the user or by the

spectrum analyzer.

Valid Range :

0 - by the user

1 - by the spectrum analyzer

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Attenuation to Read) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_DetectionMode

ViStatus AVPA_Read_DetectionMode (ViSession instrumentHandle,

ViInt16 *detectorModeStatus,

ViChar detectorMode[]);

Purpose

This function is used to read the current Trace Detector mode.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

detectorModeStatus

Variable Type       ViInt16 (passed by reference)

receives the Detector Mode status.

Valid Range :

0 - Normal

1 - Positive

2 - Negative

3 - Sample

detectorMode

Variable Type       ViChar[]

returns the Detector Mode : Posi-Nega, Positive, Negative or Sample.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_DisplayLine

ViStatus AVPA_Read_DisplayLine (ViSession instrumentHandle,

ViReal64 *levelValue,

ViInt16 *displayLineStatus,

ViChar displayLineMessage[]);

Purpose

This function returns the position and the status of the display line.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

levelValue

Variable Type       ViReal64 (passed by reference)

Returns the current setting value of the Display Line level.

Note : the unit of the Display Line level is the reference unit.

displayLineStatus

Variable Type       ViInt16 (passed by reference)

receives the Display Line status.

Valid Range :

0 - inactive

1 - active

displayLineMessage

Variable Type       ViChar[]

receives the message corresponding to the status.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_FreqCounter

ViStatus AVPA_Read_FreqCounter (ViSession instrumentHandle,

ViReal64 *counterFrequencyValueHz,

ViInt16 *frequencyCounterStatus);

Purpose

This function reads the counter frequency value and its status.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

counterFrequencyValueHz

Variable Type       ViReal64 (passed by reference)

If the counter is Active, you can read its frequency value expressed

in Hz. Otherwise, you read the frequency of the marker normal on

which the counter is placed.

Note : when you execute this front panel, double click on this

control to obtain the frequency of the marker with the chosen

accuracy.

frequencyCounterStatus

Variable Type       ViInt16 (passed by reference)

receives the Frequency Counter status.

Valid Range :

0 - Inactive

1 - Active

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_FreqSettings

ViStatus AVPA_Read_FreqSettings (ViSession instrumentHandle,

ViInt16 frequencyParameter,

ViReal64 *frequencyValueHz);

Purpose

This function is used to read the currently set value of one of these

frequency parameters : center frequency, frequency span, start frequency

or stop frequency.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyParameter

Variable Type       ViInt16

Selects the frequency parameter whose setting must be queried.

Valid Range :

0 - Center Frequency (Default)

1 - Frequency Span

2 - Start Frequency

3 - Stop Frequency

frequencyValueHz

Variable Type       ViReal64 (passed by reference)

Returns the current setting value of the selected frequency parameter

expressed in Hz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Frequency Parameter) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_FreqStepSize

ViStatus AVPA_Read_FreqStepSize (ViSession instrumentHandle,

ViReal64 *frequencyStepSizeHz,

ViInt16 *controlStatus);

Purpose

This function is used to read the Frequency Step Size expressed in Hz and

the status of the Frequency Step Size Control (Automatic or Manual)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyStepSizeHz

Variable Type       ViReal64 (passed by reference)

Returns the Frequency Step Size expressed in Hz

controlStatus

Variable Type       ViInt16 (passed by reference)

returns the status of the Frequency Step Size control.

Valid Range :

0 - Manual

1 - Automatic

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_FrequencyOffset

ViStatus AVPA_Read_FrequencyOffset (ViSession instrumentHandle,

ViReal64 *frequencyOffsetHz,

ViInt16 *frequencyOffsetStatus);

Purpose

This function is used to read the value (expressed in Hz) and the status

(Active or Inactive) of the frequency offset.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyOffsetHz

Variable Type       ViReal64 (passed by reference)

Returns the value of the Frequency Offset expressed in Hz

frequencyOffsetStatus

Variable Type       ViInt16 (passed by reference)

This indicator is used to know whether the frequency offset is active

or not.

Valid Range :

0 - inactive

1 - active

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_GatedSweep

ViStatus AVPA_Read_GatedSweep (ViSession instrumentHandle,

ViInt16 *gatedSweepStatus,

ViChar gatedSweepStatusText[],

ViReal64 *gatePositionsec,

ViReal64 *gateWidthsec,

ViInt16 *gateSourceStatus,

ViChar gateSourceType[]);

Purpose

This functions returns the features of the Gated Sweep mode :

Active/Inactive, position and width of the gate and type of trigger used.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

gatedSweepStatus

Variable Type       ViInt16 (passed by reference)

returns the Gated Sweep Status.

Valid Range :

0 - Inactive

1 - Active

gatedSweepStatusText

Variable Type       ViChar[]

returns the Gated Sweep Status : Active or Inactive.

gatePositionsec

Variable Type       ViReal64 (passed by reference)

returns the position of the gate expressed in seconds.

gateWidthsec

Variable Type       ViReal64 (passed by reference)

returns the width of the gate expressed in seconds.

gateSourceStatus

Variable Type       ViInt16 (passed by reference)

returns the type of trigger used.

Valid Range :

0 - IF signal

1 - EXT Gate In

2 - External Trigger

gateSourceType

Variable Type       ViChar[]

returns the type of trigger used : IF signal, EXT Gate In or External

Trigger.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_LevelCorrectionStatus

ViStatus AVPA_Read_LevelCorrectionStatus (ViSession instrumentHandle,

ViInt16 *levelCorrectionStatus,

ViChar levelCorrectionMessage[]);

Purpose

This function returns whether the level correction function is actually

used or not.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

levelCorrectionStatus

Variable Type       ViInt16 (passed by reference)

receives the level correction status.

Valid Range :

0 - inactive

1 - active

levelCorrectionMessage

Variable Type       ViChar[]

returns the message corresponding to the status

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_LimitLineOptions

ViStatus AVPA_Read_LimitLineOptions (ViSession instrumentHandle,

ViBoolean levelAxistoCheck,

ViInt16 *levelAxis,

ViChar levelAxismessage[],

ViReal64 *shiftLevel,

ViReal64 *frequencyTimeShift);

Purpose

This function returns the modes (absolute/relative) used along the axis

and the values of the shift level and the shift time or frequency.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

levelAxistoCheck

Variable Type       ViBoolean

this binary switch is used to select whether you want to check the

status of the Level/X-Axis or the status of the Level/Y-Axis

Valid Range :

VI_ON : X-Axis

VI_OFF : Y-Axis (Default)

levelAxis

Variable Type       ViInt16 (passed by reference)

this output variable is used to know whether we use the absolute or

the relative mode along the X or Y Axis.

Valid Range :

0 - absolute

1 - relative

levelAxismessage

Variable Type       ViChar[]

this output variable is used to know whether we use the absolute or

the relative mode along the X or Y Axis

shiftLevel

Variable Type       ViReal64 (passed by reference)

returns the value of the shift level always expressed in dB

frequencyTimeShift

Variable Type       ViReal64 (passed by reference)

returns the value of the frequency or time shift according to the

span value. If span is not null, the unit is Hz, otherwise the unit

is second.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Level/Axis to Check) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_LimitLineStatus

ViStatus AVPA_Read_LimitLineStatus (ViSession instrumentHandle,

ViBoolean limitLinetoCheck,

ViInt16 *limitLineStatus,

ViChar limitLineMessage[]);

Purpose

this function returns the status of the two limit lines.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

limitLinetoCheck

Variable Type       ViBoolean

This binary switch is used to select the limit line of which you want

to check the status.

limitLineStatus

Variable Type       ViInt16 (passed by reference)

returns whether the Limit Line is active or inactive.

Valid Range :

0 - inactive

1 - active

limitLineMessage

Variable Type       ViChar[]

returns the message corresponding to the limit line status : active

or inactive.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Limit Line to Check) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_LLjudgment

ViStatus AVPA_Read_LLjudgment (ViSession instrumentHandle,

ViInt16 *judgementStatus, ViChar judgement[]);

Purpose

this function returns the result of the limit line test.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

judgementStatus

Variable Type       ViInt16 (passed by reference)

returns the limit line judgement.

Valid Range :

0 - PASS

1 - UPPER FAIL

2 - LOWER FAIL

3 - UPPER & LOWER FAIL

judgement

Variable Type       ViChar[]

returns the result of the limit line judgement: PASS, UPPER FAIL,

LOWER FAIL or UPPER & LOWER FAIL.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_MarkerDelta

ViStatus AVPA_Read_MarkerDelta (ViSession instrumentHandle,

ViReal64 *markerDeltaPosition,

ViReal64 *markerDeltaLevel);

Purpose

This function is used to read the position and the level of the delta

marker.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

markerDeltaPosition

Variable Type       ViReal64 (passed by reference)

returns the position of the marker delta expressed in Hz when span is

not null and in second when span is null.

markerDeltaLevel

Variable Type       ViReal64 (passed by reference)

returns the level of the marker delta without unit.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_MarkerDeltaOptions

ViStatus AVPA_Read_MarkerDeltaOptions (ViSession instrumentHandle,

ViInt16 *fixedMarkerStatus,

ViChar fixedMarkerMessage[],

ViInt16 *displayModeStatus,

ViChar displayMode[]);

Purpose

This function allows to know the display mode and to know whether the

marker delta is fixed or not.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

fixedMarkerStatus

Variable Type       ViInt16 (passed by reference)

receives the fixed marker delta status :

Valid Range :

0 - inactive

1 - active

fixedMarkerMessage

Variable Type       ViChar[]

returns the message corresponding to the Fixed Marker status.

displayModeStatus

Variable Type       ViInt16 (passed by reference)

receives the display mode status.

Valid Range :

0 - absolute

1 - relative

displayMode

Variable Type       ViChar[]

returns the display mode : absolute or relative.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_MarkerNormal

ViStatus AVPA_Read_MarkerNormal (ViSession instrumentHandle,

ViReal64 *markerNormalPosition,

ViReal64 *markerNormalLevel);

Purpose

This function returns the position and the level of the Marker Normal.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

markerNormalPosition

Variable Type       ViReal64 (passed by reference)

returns the position of the marker normal expressed in Hz when span

is not null and in second when span is null.

markerNormalLevel

Variable Type       ViReal64 (passed by reference)

returns the level of the marker normal without unit.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_MarkerStatus

ViStatus AVPA_Read_MarkerStatus (ViSession instrumentHandle,

ViInt16 *markerStatus,

ViChar markerStatusMessage[]);

Purpose

This function is used to know whether the markers normal and delta are

active or not.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

markerStatus

Variable Type       ViInt16 (passed by reference)

receives the marker status.

Valid Range :

0 - Markers Normal and Delta inactive

1 - Only Marker Normal active

2 - Markers Normal and Delta active

markerStatusMessage

Variable Type       ViChar[]

returns the marker status message corresponding to the marker status

number : "Markers Normal and Delta inactive", "Only Marker Normal

active" or "Markers Normal and Delta active".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_MarkerTraceStatus

ViStatus AVPA_Read_MarkerTraceStatus (ViSession instrumentHandle,

ViInt16 *markerTraceStatus,

ViChar markerTraceMessage[]);

Purpose

this function returns the name of the trace where the marker is displayed

(active).

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

markerTraceStatus

Variable Type       ViInt16 (passed by reference)

receives the marker trace status (which trace is the marker pointing

to ?).

Valid Range :

0 - Blank

1 - Trace A

2 - Trace B

markerTraceMessage

Variable Type       ViChar[]

returns the message corresponding to the status

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_MeasurementWindow

ViStatus AVPA_Read_MeasurementWindow (ViSession instrumentHandle,

ViReal64 *windowPosition,

ViReal64 *windowWidth,

ViInt16 *windowStatus,

ViChar windowStatusMessage[]);

Purpose

This function is used to read the features of the measurement window and

to know whether the Measurement window is active or not.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

windowPosition

Variable Type       ViReal64 (passed by reference)

position of the Measurement Window expressed in Hz when span is not

null and in second when span is null

windowWidth

Variable Type       ViReal64 (passed by reference)

Width of the Measurement Window expressed in Hz when span is not null

and in second when span is null.

windowStatus

Variable Type       ViInt16 (passed by reference)

receives the status of the measurement window.

Valid Range :

0 - active

1 - inactive

windowStatusMessage

Variable Type       ViChar[]

returns the message according to the status number

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_MultiMarkerActive

ViStatus AVPA_Read_MultiMarkerActive (ViSession instrumentHandle,

ViInt16 *multiMarkerActive);

Purpose

This function returns the index of the active Multi-Marker

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

multiMarkerActive

Variable Type       ViInt16 (passed by reference)

returns the index of the Multi-Marker active

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_Multimarkers

ViStatus AVPA_Read_Multimarkers (ViSession instrumentHandle,

ViReal64 multimarkersPositions[],

ViReal64 *markerDeltaPosition,

ViReal64 multimarkersLevels[],

ViReal64 *markerDeltaLevel);

Purpose

This function is used to read the positions and levels of the 10

multimarkers and the position and level of the marker delta.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

multimarkersPositions

Variable Type       ViReal64[]

this array contains either the positions of the multimarkers when the

marker delta is inactive or the differences of position  between the

multimarkers and the marker delta when this last one is active.

When the Multi Marker x is not active, you read zero.

markerDeltaPosition

Variable Type       ViReal64 (passed by reference)

returns the position of the marker delta when it is active and zero

otherwise.

multimarkersLevels

Variable Type       ViReal64[]

this array contains :

- the levels of the multimarkers when marker delta is inactive.

- the absolute levels of the multimarkers when the marker delta  is

active and display mode is set on absolute.

- the level gaps between the multimarkers and the marker delta when

it is active and display mode set on relative.

When the Multi Marker x is not active, you read zero.

markerDeltaLevel

Variable Type       ViReal64 (passed by reference)

returns the level of the marker delta when it is active and zero

otherwise.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_MultimarkersStatus

ViStatus AVPA_Read_MultimarkersStatus (ViSession instrumentHandle,

ViInt16 *multimarkersStatus,

ViChar multimarkersMessage[]);

Purpose

This function allows to know whether the multimarkers are active or  not.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

multimarkersStatus

Variable Type       ViInt16 (passed by reference)

receives the Multimarkers status.

Valid Range :

0 - inactive

1 - active

multimarkersMessage

Variable Type       ViChar[]

returns the message corresponding to the status.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_Peak_Options

ViStatus AVPA_Read_Peak_Options (ViSession instrumentHandle,

ViInt16 *continuousPeakStatus,

ViChar continuousPeakMessage[],

ViInt16 *signalTrackStatus,

ViChar signalTrackMessage[],

ViInt16 *coupledtoWindowStatus,

ViChar coupledToWindowMessage[],

ViReal64 *peakDelta_Y);

Purpose

This function is used to read the continuous peak status, the signal

track status, the coupled to window status and the value of Peak Delta_Y.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

continuousPeakStatus

Variable Type       ViInt16 (passed by reference)

receives the continuous peak status.

Valid Range :

0 - OFF

1 - ON

continuousPeakMessage

Variable Type       ViChar[]

receives the message corresponding to the continuous peak status.

signalTrackStatus

Variable Type       ViInt16 (passed by reference)

receives the signal track status.

Valid Range :

0 - OFF

1 - ON

signalTrackMessage

Variable Type       ViChar[]

receives the message corresponding to the signal track status.

coupledtoWindowStatus

Variable Type       ViInt16 (passed by reference)

receives the coupled to window status .

Valid Range :

0 - inactive

1 - active

coupledToWindowMessage

Variable Type       ViChar[]

receives the message corresponding to the "coupled to window" status.

peakDelta_Y

Variable Type       ViReal64 (passed by reference)

returns the value of Peak Delta_Y

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_RBWandVBW

ViStatus AVPA_Read_RBWandVBW (ViSession instrumentHandle,

ViBoolean bandwidthtoRead,

ViReal64 *bandwidthValueHz,

ViInt16 *typeofControl);

Purpose

This function returns the values of the resolution and video bandwidths.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

bandwidthtoRead

Variable Type       ViBoolean

selects the bandwidth that you want to read.

Valid Range :

VI_ON - RBW (Default)

VI_OFF - VBW

bandwidthValueHz

Variable Type       ViReal64 (passed by reference)

returns the value of the Resolution Bandwidth or of the Video

Bandwidth always expressed in Hz

typeofControl

Variable Type       ViInt16 (passed by reference)

returns whether the RBW or the VBW is set up by the user or by the

spectrum analyzer.

Valid Range :

0 - by the user

1 - by the spectrum analyzer

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Bandwidth to Read) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_ReferenceLevel

ViStatus AVPA_Read_ReferenceLevel (ViSession instrumentHandle,

ViReal64 *referenceLevelValue);

Purpose

This function is used to read the value (without unit) of the  Reference

Level.

Note : the unit is returned by the function "Read Reference Unit".

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceLevelValue

Variable Type       ViReal64 (passed by reference)

Returns the current setting value of the Reference Level.

Note : the unit is returned by the function "Read Reference Unit".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_RefLevelOffset

ViStatus AVPA_Read_RefLevelOffset (ViSession instrumentHandle,

ViReal64 *referenceLevelOffsetValue,

ViInt16 *typeofControl);

Purpose

This function is used to read the value (the unit is always dB) and the

status of the  Reference Level offset.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceLevelOffsetValue

Variable Type       ViReal64 (passed by reference)

Returns the current setting value of the Reference Level Offset (unit

is always dB).

typeofControl

Variable Type       ViInt16 (passed by reference)

This indicator is used to know whether the reference level offset is

active or not.

Valid Range :

0 - inactive

1 - active

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_RefUnit

ViStatus AVPA_Read_RefUnit (ViSession instrumentHandle,

ViInt16 *referenceUnitNumber,

ViChar referenceUnitName[]);

Purpose

This function returns the name and the number corresponding to the name

of the Reference Unit.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceUnitNumber

Variable Type       ViInt16 (passed by reference)

Returns the number corresponding to the Reference Unit

Valid Range :

0 - dBm

1 - dBmV

2 - dBuV

3 - dBuVemf

4 - dBpW

6 - V

7 - W

referenceUnitName

Variable Type       ViChar[]

Returns the name of the Reference Unit : dBm, dBmV, dBuV, dBuVemf,

dBpW, V or W

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_Scale

ViStatus AVPA_Read_Scale (ViSession instrumentHandle, ViInt16 *scaleStatus,

ViChar nameoftheScale[]);

Purpose

This function returns the name of the scale and its status.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

scaleStatus

Variable Type       ViInt16 (passed by reference)

receives the scale status :

if log mode :

0 - 10 dB

1 - 5 dB

2 - 2 dB

3 - 1 dB

4 - 0.5 dB

if linear mode :

5 - Linx 1

6 - Linx 2

7 - Linx 5

8 - Linx 10

nameoftheScale

Variable Type       ViChar[]

returns the name of the scale : 10 dB, 5 dB, 2 dB, 1 dB, 0.5 dB, Lin

x1, Lin x2, Lin x5 or Lin x10

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_StatusByte

ViStatus AVPA_Read_StatusByte (ViSession instrumentHandle,

ViInt16 *statusByteRead);

Purpose

This function is used to read the status byte register.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

statusByteRead

Variable Type       ViInt16 (passed by reference)

returns the Status Byte.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_SweepMode

ViStatus AVPA_Read_SweepMode (ViSession instrumentHandle,

ViInt16 *sweepModeStatus, ViChar sweepMode[]);

Purpose

This function returns the Sweep Mode and says whether the sweep is

performed in the full screen or only in the Measurement Window.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sweepModeStatus

Variable Type       ViInt16 (passed by reference)

returns the Sweep Mode status.

Valid Range :

0 - Normal & full

20 - Single & full

1 - Normal & window

21 - Single & window

Notes :

(1) Normal - Continuous sweep

(2) Single - single sweep

(3) Full - measurement window inactive

(4) Window - measurement window active

sweepMode

Variable Type       ViChar[]

returns the Sweep Mode and says whether the sweep is performed in the

full screen or only in the Measurement Window.

Valid Range :

Normal & full

Single & full

Normal & window

Single & window

Notes :

(1) Normal - Continuous sweep

(2) Single - single sweep

(3) Full - measurement window inactive

(4) Window - measurement window active

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_SweepTime

ViStatus AVPA_Read_SweepTime (ViSession instrumentHandle,

ViReal64 *timeValuesecond, ViInt16 *typeofControl);

Purpose

This function is used to read the Sweep Time value expressed always in

seconds and the status of the sweep time control (manual or automatic)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

timeValuesecond

Variable Type       ViReal64 (passed by reference)

Returns the current setting value of the Sweep Time expressed always

in second.

typeofControl

Variable Type       ViInt16 (passed by reference)

returns whether the Sweep Time is set up by the user or by the

spectrum analyzer.

Valid Range :

0 - by the user

1 - by the spectrum analyzer

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_TraceData

ViStatus AVPA_Read_TraceData (ViSession instrumentHandle, ViBoolean tracetoRead,

ViBoolean dataFormat, ViInt16 traceDataArray[]);

Purpose

This function is used to read a trace. The result is send back in the

trace Data Array.

Beware : it is impossible to read the trace when a RS-232 link is used.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

tracetoRead

Variable Type       ViBoolean

selects the trace to store in an array.

Valid Range :

VI_ON - Trace A (Default)

VI_OFF - Trace B

dataFormat

Variable Type       ViBoolean

Selects the transfer format.

Valid Range :

VI_ON - Binary (Default)

VI_OFF - ASCII

traceDataArray

Variable Type       ViInt16[]

This is the array where the function will write the trace data.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace to Read) out of range.

BFFC0003  Parameter 3 (Data Format) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_TracePoints

ViStatus AVPA_Read_TracePoints (ViSession instrumentHandle,

ViInt16 *numberofPointsinTrace);

Purpose

This function is used to read the number of points in the trace.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberofPointsinTrace

Variable Type       ViInt16 (passed by reference)

returns the number of points in trace : 501 or 1001 points.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_TraceStatus

ViStatus AVPA_Read_TraceStatus (ViSession instrumentHandle,

ViBoolean traceModetoRead,

ViInt16 *traceModeStatus,

ViChar generalStatusText[]);

Purpose

This function returns the trace mode of the Trace A and The Trace B

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

traceModetoRead

Variable Type       ViBoolean

selects the name of the trace to scan

Valid Range :

VI_ON - Trace A (Default)

VI_OFF - Trace B

traceModeStatus

Variable Type       ViInt16 (passed by reference)

receives the trace mode status

Valid Range :

TRACE A :

---------

lower byte :

0  -  write

1  -  view

2  -  blank

3  -  A-DL->A

4  -  A-B->A

5  -  B-A->A

upper byte :

0  -  nothing

1  -  + max hold

2  -  + averaging

3  -  + min hold

TRACE B :

---------

lower byte :

1  -  view

2  -  blank

upper byte :

0  -  nothing

generalStatusText

Variable Type       ViChar[]

returns the mode corresponding to the status displayed in the other

output variable.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace Mode to Read) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Read_TriggerMode

ViStatus AVPA_Read_TriggerMode (ViSession instrumentHandle,

ViInt16 *triggerModeStatus,

ViChar triggerMode[],

ViInt16 *levelofVideoTrigger);

Purpose

This function returns the Trigger Mode and the level of the video

trigger.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerModeStatus

Variable Type       ViInt16 (passed by reference)

returns the Trigger Mode status.

Valid Range :

0 - Free Run

1 - Line

2 - Video

5 - External

triggerMode

Variable Type       ViChar[]

returns the Trigger Mode : Free Run, Line, Video or External.

levelofVideoTrigger

Variable Type       ViInt16 (passed by reference)

returns the level of the Video Trigger.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Recall_File

ViStatus AVPA_Recall_File (ViSession instrumentHandle, ViBoolean recallType,

ViInt16 registerName, ViString fileName);

Purpose

This function is used to recall a register or a file.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

recallType

Variable Type       ViBoolean

selects whether you want recall a register or a file

Valid Range :

VI_ON - File

VI_OFF - Register (Default)

registerName

Variable Type       ViInt16

selects the name of the register to recall

Valid Range :

1 - REG_01 (Default)

2 - REG_02

3 - REG_03

4 - REG_04

5 - REG_05

6 - REG_06

7 - REG_07

8 - REG_08

9 - REG_09

10 - REG_10

fileName

Variable Type       ViString

Contains the name (8 characters maxi) of the file to recall.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Recall Type) out of range.

BFFC0003  Parameter 3 (Register Name) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_reset

ViStatus AVPA_reset (ViSession instrumentHandle);

Purpose

This function resets the instrument to a known state and sends

initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_revisionQuery

ViStatus AVPA_revisionQuery (ViSession instrumentHandle,

ViChar instrumentDriverRevision[],

ViChar firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware, and tells the user with which  instrument firmware

this revision of the driver is compatible.

For example :

This instrument driver's Revision Number is "Rev 1.0, 02/98, CVI 4.0.1"

and is compatible with Instrument Firmware Revision H03.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

This control returns the Instrument Driver Software Revision.

firmwareRevision

Variable Type       ViChar[]

This control returns the Instrument Firmware Revision.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Save_File

ViStatus AVPA_Save_File (ViSession instrumentHandle, ViBoolean saveType,

ViInt16 registerName, ViString fileName);

Purpose

This function is used to save data into a file or a register.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

saveType

Variable Type       ViBoolean

This binary switch is used to choose between Register or File to save

the data

Valid Range :

VI_ON - File

VI_OFF - Register (Default)

registerName

Variable Type       ViInt16

selects the name of the register into save the data

Valid Range :

1 - REG_01 (Default)

2 - REG_02

3 - REG_03

4 - REG_04

5 - REG_05

6 - REG_06

7 - REG_07

8 - REG_08

9 - REG_09

10 - REG_10

fileName

Variable Type       ViString

Contains the name (8 characters maxi) of the file into save the data.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Save Type) out of range.

BFFC0003  Parameter 3 (Register Name) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Search_Peak

ViStatus AVPA_Search_Peak (ViSession instrumentHandle, ViInt16 typeofSearch);

Purpose

This function performs different type of peak marker search.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofSearch

Variable Type       ViInt16

selects the peak marker search desired

Valid Range :

0 - peak search (Default)

1 - next peak

2 - next peak left

3 - next peak right

4 - min search

5 - next minimum

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Search) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Select_MemoryDrive

ViStatus AVPA_Select_MemoryDrive (ViSession instrumentHandle,

ViInt16 memoryDrive);

Purpose

This fonction is used to select the memory drive you want to use between

the drive A, the drive B or the RAM memory.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

memoryDrive

Variable Type       ViInt16

This controls allows you to select the memory device you want to use.

Valid Range :

1 - Drive A (Memory Card A)

2 - Drive B (Memory Card B)

3 - RAM (RAM Memory)

Default : 1 - Drive A

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Memory Card Drive) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_selfTest

ViStatus AVPA_selfTest (ViSession instrumentHandle, ViInt16 *selfTestResult,

ViChar selfTestMessage[]);

Purpose

This function runs the instrument's self test routine and returns the

test result(s).

This instrument does not support a Self-Test, this function returns the

Warning Code 0x3FFC0103 - VI_WARN_NSUP_SELF_TEST

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

selfTestResult

Variable Type       ViInt16 (passed by reference)

This control contains the value returned from the instrument self

test.  Self-Test is not supported by this instrument this control

will always return negative one (-1).

selfTestMessage

Variable Type       ViChar[]

This control contains the string returned from the self test. This

instrument does not support a self-test and this control will always

return the message "Self-Test Not Supported".

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFC0103  Self Test not supported - VI_WARN_NSUP_SELF_TEST

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_ActiveMultiMarker

ViStatus AVPA_Set_ActiveMultiMarker (ViSession instrumentHandle,

ViInt16 multiMarkerActive);

Purpose

This function is used to choose the Multi-Marker that will be the active

Multi-Marker.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

multiMarkerActive

Variable Type       ViInt16

This slide is used to select the MultiMarker that will be the

active Multi-Marker.

Valid Range :

1 - 1  (Default)

2 - 2

3 - 3

4 - 4

5 - 5

6 - 6

7 - 7

8 - 8

9 - 9

10 - 10

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Multi Marker Active) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFC0810  VI_ERROR_MARKERINACTIVE Multimarker chosen is inactive

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_All_Auto_Couple

ViStatus AVPA_Set_All_Auto_Couple (ViSession instrumentHandle);

Purpose

This function sets all coupled functions (RBW, VBW, and Sweep Time) to

AUTO, according to the current SPAN.

No parameter is needed.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_Att

ViStatus AVPA_Set_Att (ViSession instrumentHandle, ViBoolean typeofControl,

ViReal64 inputAttenuationValuedB);

Purpose

This function is used to configure the input attenuator. Normal (default)

position is AUTO. This is also the safest position.

Note : the input attenuation setting is conditioned by the minimum

attenuation setting: a value smaller than the minimum attenuation cannot

be set. This limitation is controlled by the analyzer itself.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofControl

Variable Type       ViBoolean

This Binary Switch allows to control the input attenuation manually

or to let the spectrum analyser choose its value.

Valid Range:

VI_OFF - Manual

VI_ON - Automatic ( Default)

Note : the other parameter of this function is "Input Attenuation

Value" which is only valid when "VI_OFF-Manual" is selected by this

control.

inputAttenuationValuedB

Variable Type       ViReal64

Value that the input attenuation should be set to.

Data Range: 0 to 70 dB

Default Value : 10 dB

Warning: the analyzer will not set input attenuation under the

minimum attenuation setting. If a value less than the minimum is set

here the minimum attenuation will be set instead.

Note : the Input Attenuation is always expressed in dB.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Control) out of range.

BFFC0003  Parameter 3 (Input Attenuation Value) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_DisplayLine

ViStatus AVPA_Set_DisplayLine (ViSession instrumentHandle,

ViBoolean showDisplayLine,

ViReal64 displayLinePosition);

Purpose

This function is used to define the display line position and show or

hide the display line.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

showDisplayLine

Variable Type       ViBoolean

This binary switch is used to show or hide the display line.

Valid Range :

VI_ON - Show

VI_OFF - Hide (Default)

displayLinePosition

Variable Type       ViReal64

Specifies the position where the display line should be shown.

Valid Range : -1000 to +1000

Note : This position is expressed in the same unit as the one defined

by the function Set Reference Unit.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Show Display Line) out of range.

BFFC0003  Parameter 3 (Display Line Position) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_Freq

ViStatus AVPA_Set_Freq (ViSession instrumentHandle, ViInt16 frequencyType,

ViReal64 frequencyValueHz);

Purpose

This function sets the specified frequency (Frequency Type) to the

specified value (Frequency value).

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyType

Variable Type       ViInt16

The Frequency Type determines which frequency parameter must be set

between the center frequency and the start or stop frequency.

Valid Range :

0 - Center (Default)

1 - Start

2 - Stop

frequencyValueHz

Variable Type       ViReal64

The frequency value to be set for either the center, start or stop

frequency depending on the Frequency Type setting.

Center frequency valid range :

0 to 3 GHz for the R3263 and R3463

0 to 8 GHZ for the R3465

0 to 26.5 GHz for the R3272

Start frequency valid range :

-200 MHz to 3 GHz for the R3263 and R3463

-200 MHz to 8 GHZ for the R3465

-200 MHz to 26.5 GHz for the R3272

Stop frequency valid range :

0 to 3.2 GHz for the R3263 and R3463

0 to 8.2 GHZ for the R3465

0 to 26.7 GHz for the R3272

Default Value: 1 GHz

Note : the frequency is always expressed in Hz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Frequency Type) out of range.

BFFC0003  Parameter 3 (Frequency Value) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_FreqCounter

ViStatus AVPA_Set_FreqCounter (ViSession instrumentHandle,

ViBoolean counterActivation,

ViInt16 counterResolution);

Purpose

This function is used to activate/desactivate the frequency counter and

to selects its resolution.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

counterActivation

Variable Type       ViBoolean

This binary switch is used to activate or desactivate the frequency

counter.

Valid Range :

VI_ON - ON

VI_OFF - OFF (Default)

counterResolution

Variable Type       ViInt16

This slide is used to select the frequency counter's resolution

Valid Range:

0 - 1 kHz  (Default)

1 - 100 Hz

2 - 10 Hz

3 - 1 Hz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Counter Activation) out of range.

BFFC0003  Parameter 3 (Counter Resolution) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_FreqOffset

ViStatus AVPA_Set_FreqOffset (ViSession instrumentHandle,

ViBoolean frequencyOffsetActivation,

ViReal64 frequencyOffsetHz);

Purpose

This fonction is used to set up the Frequency Offset

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyOffsetActivation

Variable Type       ViBoolean

This binary switch allows to activate or desactivate the frequency

offset.

Valid Range :

VI_ON : Active

VI_OFF : Inactive (Default)

frequencyOffsetHz

Variable Type       ViReal64

Value of the frequency offset.

Valid Range: 0 (Default) to +/-100 MHz

Default Value: 0 Hz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Freq Offset Activation) out of range.

BFFC0003  Parameter 3 (Frequency Offset) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_FreqStepSize

ViStatus AVPA_Set_FreqStepSize (ViSession instrumentHandle,

ViBoolean typeofControl,

ViReal64 frequencyStepHz);

Purpose

This function is used to set up the frequency step size.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofControl

Variable Type       ViBoolean

This binary switch lets the user choose whether he determines himself

the value of the frequncy step or he lets the spectrum analyzer

choose this parameter

Data range:

VI_ON - Auto (Default)

VI_OFF - Manual

Note : In Auto mode, the spectrum analyzer automatically adjust the

step size to 10% of the center frequency.

frequencyStepHz

Variable Type       ViReal64

Value of the frequency step size (center frequency setting).

Valid Range:

0 to 3 GHz for the R3263 and R3463

0 to 8 GHz for the R3465

0 to 26.5 GHz for the R3272

Default Value : 0 Hz

Note : This control is only valid when "Type of control" is on

"Manual".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Control) out of range.

BFFC0003  Parameter 3 (Frequency Step) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_GatedSweep

ViStatus AVPA_Set_GatedSweep (ViSession instrumentHandle, ViBoolean gatedSweep,

ViInt16 gateSource, ViBoolean gateSourceSlope,

ViReal64 gatePositionsec, ViReal64 gateWidthsec);

Purpose

This function is used to configure and activate or desactivate the Gated

Sweep Mode

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

gatedSweep

Variable Type       ViBoolean

Activate or desactivate the Gated Sweep mode.

Valid Range:

VI_OFF - OFF (Default)

VI_ON - ON

Note: If trigger signal is incorrect the analyzer might then fall

into an endless wait loop.

gateSource

Variable Type       ViInt16

Selects the gate source ie the signal used as a trigger for

positionning the gate.

Data Range:

0 - IF Signal (Default Value)

1 - External Gate IN

2 - External Trigger

Notes :

(1) IF Signal : Internal IF signal

(2) External Gate IN : Gate signal connected to the GATE IN

connector at the rear panel of analyzer

(3) External Trigger : Pulse TTL signal connected to the

external trigger connector at the rear panel of the analyzer

gateSourceSlope

Variable Type       ViBoolean

Selects the polarity of the external trigger signal

Data Range:

VI_OFF : -Slope

VI_ON : +Slope (Default)

gatePositionsec

Variable Type       ViReal64

Gate Position: Time at which the gate should start.

Data Range : 1e-6 to 65e-3 sec

Default Value : 1e-6 sec

gateWidthsec

Variable Type       ViReal64

Gate width: size of the gate. Gate starts at Gate Position and ends

at Gate Position + Gate Width.

Data Range : 2e-6 to 65e-3 sec

Default Value : 5e-3 sec

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Gated Sweep) out of range.

BFFC0003  Parameter 3 (Gate Source) out of range.

BFFC0004  Parameter 4 (Gate Source Slope) out of range.

BFFC0005  Parameter 5 (Gate Position) out of range.

BFFC0006  Parameter 6 (Gate Width) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_LevelCorrection

ViStatus AVPA_Set_LevelCorrection (ViSession instrumentHandle,

ViBoolean levelCorrection,

ViString levelCorrectionFrequencyTable,

ViString levelCorrectionLevelTable);

Purpose

This function is used to activate or desactivate the level correction

function and to set up the table of level correction.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

levelCorrection

Variable Type       ViBoolean

specifies whether the input level correction is active (ON) or

inactive (OFF).

Valid Range:

VI_ON - ON

VI_OFF - OFF (Default)

levelCorrectionFrequencyTable

Variable Type       ViString

Frequencies table of the correction factor function.

Valid Range : depends on the frequency range studied.

Syntax : "f1,f2,f2,f3"

Note : only the frequency domain is available and the unit is always

Hz.

levelCorrectionLevelTable

Variable Type       ViString

Levels table of the correction factor function.

Valid Range : depends on the level range studied.

Syntax : "l1,l1,l2,l1"

Note : the unit is always the dB.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Level Correction) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_LimitLine

ViStatus AVPA_Set_LimitLine (ViSession instrumentHandle, ViBoolean limitLine,

ViString limitLineFrequencyTimeTable,

ViString limitLineLevelTable);

Purpose

This function is used to set up the tables of the differents Limit Lines.

There are two Limit Lines called 1 and 2 and for each one two tables

(frequency domain and time domain).

Note that it is useless to specify the domain (frequency or time) because

the function check it before beginning.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

limitLine

Variable Type       ViBoolean

Selects the limit line that will be configured.

Valid Range:

VI_ON - Limit Line 1 (Default)

VI_OFF - Limit Line 2

limitLineFrequencyTimeTable

Variable Type       ViString

Frequencies table of the Limit Line which number is specified by the

control "Limit Line".

Syntax : "f1,f2,f2,f3,f3,f4"

Notes :

(1) If span is not null, it will be the frequency domain's

table.Otherwise, it will be the time domain's table.

(2) Units : Frequency in Hz and Time in second.

limitLineLevelTable

Variable Type       ViString

Levels table of the Limit Line which number is specified by the

control "Limit Line".

Syntax : "l1,l1,l2,l2,l3,l3"

Note : Unit of level is always dB.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Limit Line) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_MarkerDelta

ViStatus AVPA_Set_MarkerDelta (ViSession instrumentHandle,

ViReal64 deltaPositionHzors);

Purpose

This panel is used to activate and to set the difference of

frequency/time position between the Marker Normal and the   Marker Delta.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

deltaPositionHzors

Variable Type       ViReal64

Specifies where to position the Marker Delta : this is the difference

between the frequency/time position of the Marker Normal and the

frequency/time position of the Marker Delta.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_MarkerDeltaOptions

ViStatus AVPA_Set_MarkerDeltaOptions (ViSession instrumentHandle,

ViBoolean one1DeltaMarker,

ViBoolean markerDeltaFixed,

ViBoolean markerDisplayMode);

Purpose

This function is used to set up the options about the Marker Delta.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

one1DeltaMarker

Variable Type       ViBoolean

This binary switch is used to activate or desactivate the mode

"1/Delta Marker".

Valid Range :

VI_ON : Active

VI_OFF : Inactive (Default)

markerDeltaFixed

Variable Type       ViBoolean

This binary switch is used to activate or desactivate the mode "Fixed

Marker Delta".

Valid Range :

VI_ON : Active

VI_OFF : Inactive (Default)

markerDisplayMode

Variable Type       ViBoolean

This binary switch is used to select the Marker Display Mode.

Valid Range :

VI_ON - Relative (Default)

VI_OFF - Absolute

Notes :

(1) if RELATIVE, the analyzer returns the level's gap between the

markers Normal and Delta.

(2) if ABSOLUTE, the analyzer returns the absolute level of the

marker Normal even if the marker Delta is active.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (one (1) / Delta Marker) out of range.

BFFC0003  Parameter 3 (Marker Delta Fixed) out of range.

BFFC0004  Parameter 4 (Marker Display Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_MarkerDeltaTo

ViStatus AVPA_Set_MarkerDeltaTo (ViSession instrumentHandle,

ViInt16 markerDeltaTo);

Purpose

This function is used to set one of the following parameters: center

frequency, span, CF step and marker step, with the value of the delta

marker.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

markerDeltaTo

Variable Type       ViInt16

selects the parameter that will be assigned to the current delta

marker value.

Valid Range :

0 - Center Frequency (Default)

1 - Span

2 - CF Step

3 - Marker Step

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Marker Delta To) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_MarkerNormal

ViStatus AVPA_Set_MarkerNormal (ViSession instrumentHandle,

ViReal64 positionHzors);

Purpose

This panel is used to activate and to position the Marker Normal.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

positionHzors

Variable Type       ViReal64

Specifies where to position the Marker Normal : this is either a

frequency (Hz) or a time (sec), depending on the current span.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_MarkerTrace

ViStatus AVPA_Set_MarkerTrace (ViSession instrumentHandle,

ViBoolean markerTrace);

Purpose

This function is used to position the marker(s) on trace A  or Trace B.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

markerTrace

Variable Type       ViBoolean

this binary switch is used to position the marker(s) on trace A  or

Trace B.

Valid Range :

VI_ON - Trace A (Default)

VI_OFF - Trace B

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Marker Trace) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_MeasurementWindow

ViStatus AVPA_Set_MeasurementWindow (ViSession instrumentHandle,

ViBoolean showHideWindow,

ViReal64 windowPosition,

ViReal64 windowWidth);

Purpose

This function is used to specify the position and the width of the

Measurement Window and to activate or desactivate it.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

showHideWindow

Variable Type       ViBoolean

This binary switch is used to show or hide the Measurement Window.

Valid Range:

VI_ON - Show

VI_OFF - Hide (Default)

windowPosition

Variable Type       ViReal64

Specifies the position of the Measurement Window.

Valid Range : depends on the frequency range studied or on the sweep

time.

Default Value : 0

Notes :

(1) When span is not null, window position = position of the window's

center and unit is Hz.

(2) When span is null, window position = position of the window's

left side and unit is second.

windowWidth

Variable Type       ViReal64

Specifies the width of the Measurement Window.

Valid Range : depends on the frequency range studied or on the sweep

time.

Default Value : 0

Notes :

(1) When span is not null, unit is Hz.

(2) When span is null, unit is sec.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Show/Hide Window) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_MinATT

ViStatus AVPA_Set_MinATT (ViSession instrumentHandle, ViBoolean typeofControl,

ViReal64 minimumInputAttenuationValue);

Purpose

This function is used to select whether the Minimum Input Attenuation is

chosen manually or by the analyzer and to choose its value in the first

case.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofControl

Variable Type       ViBoolean

The minimum attenuation can be set either manually (User) or by the

spectrum analyzer (Automatic).

Valid Range :

VI_OFF - User

VI_ON - Automatic (Default)

Note : when "Automatic" is selected the Minimum Input Attenuation is

set to 10 dB.

minimumInputAttenuationValue

Variable Type       ViReal64

Specifies the minimum input attenuation that can be set when the

Type of control switch is set to "User".

Data Range: 0 to  70 dB

Default Value:  10 dB   (= value used by the analyzer when the Type

of Control selector is set to "Automatic").

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Control) out of range.

BFFC0003  Parameter 3 (Min. Input Att. Value) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_Multimarkers

ViStatus AVPA_Set_Multimarkers (ViSession instrumentHandle,

ViString multiMarkersStatus,

ViString multiMarkersPositions);

Purpose

This function is used to activate and position the number of Multimakers

that you need.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

multiMarkersStatus

Variable Type       ViString

status of the 10 multimarkers.

Valid Range :

0 : Multi-Marker inactive.

1 : Multi-Marker active.

Syntax : "0,1,1,1,0,1,1,0,1,1"

multiMarkersPositions

Variable Type       ViString

positions of the 10 multimarkers.

Syntax : "p1,p2,p3,p4,p5,p6,p7,p8,p9,p10"

Notes :

(1) - the position is always the absolute position of the multimarker

even if the marker delta is active.

(2) - if the status of the multi-marker i is equal to zero, the value

of the position is not used.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_PeakSearchOptions

ViStatus AVPA_Set_PeakSearchOptions (ViSession instrumentHandle,

ViBoolean continousPeakSearch,

ViBoolean signalTrack,

ViBoolean coupleToWindow,

ViInt16 peakRange,

ViReal64 peakdeltaYvalue);

Purpose

This function is used to determine the value of Peak delta Y, the part of

the screen where the peak will be searched and activate or desactivate

the continuous peak search and the signal track.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

continousPeakSearch

Variable Type       ViBoolean

This binary switch allows to execute a continuous search of the peak

or not

Valid Range :

VI_ON : Active

VI_OFF : Inactive (Default)

signalTrack

Variable Type       ViBoolean

This binary switch allows to use the option Signal Track.

Valid Range :

VI_ON : Active

VI_OFF : Inactive (Default)

coupleToWindow

Variable Type       ViBoolean

This binary switch allows to search the peaks only in the measurement

window.

Valid Range :

VI_ON : Active

VI_OFF : Inactive (Default)

peakRange

Variable Type       ViInt16

Selects the peak search range.

Valid Range:

0 - Normal (Default)

1 - Upper Side

2 - Lower Side

Notes :

(1) Normal : searches the next peak among all points of the waveform

(2) Upper Side : searches the next peak among the trace points

above the display line

(3) Lower Side : searches the next peak among the trace points

below the display line.

peakdeltaYvalue

Variable Type       ViReal64

This control is used to set up the value of Peak deltaY.

Valid Range: 0.1 to 10

Default Value : 1

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Continuous Peak Search) out of range.

BFFC0003  Parameter 3 (Signal Track) out of range.

BFFC0004  Parameter 4 (Couple To Window) out of range.

BFFC0005  Parameter 5 (Peak Range) out of range.

BFFC0006  Parameter 6 (Peak DeltaY Value) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_RBW

ViStatus AVPA_Set_RBW (ViSession instrumentHandle, ViInt16 RBWSetting);

Purpose

This function is used to set up the bandwidth of the filter of

resolution.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

RBWSetting

Variable Type       ViInt16

This control is used to specify the value of the bandwidth of the

filter of resolution (RBW).

Valid Range:

0 - AUTO    (Default)

1 - 200 Hz

2 - 300 Hz

3 - 1 kHz

4 - 3 kHz

5 - 10 kHz

6 - 30 kHz

7 - 100 kHz

8 - 300 kHz

9 - 1 MHz

10 - 3 MHz

11 - 5 MHz

Notes :

(1) Automatic RBW can be selected in which case the analyzer    will

automatically choose the RBW that fits best the current SPAN

selection.

(2) The ratio RBW:Span can be modified using the function "Set

RBW/Span coupling".

(3) When "200 Hz" is selected, band accuracy, level accuracy and

noise level are no longer regulated.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (RBW Setting) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_RBW_Span

ViStatus AVPA_Set_RBW_Span (ViSession instrumentHandle, ViBoolean typeofControl,

ViReal64 ratioRBWto1Span);

Purpose

This function is used to set up the ratio between RBW and the span. This

ratio is used to estimate the appropriate setting for RBW when RBW->AUTO

is selected (cf function "Set RBW").

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofControl

Variable Type       ViBoolean

Enables manual setting of the ratio RBW to Span. Resets the RBW /

Span coupling to its default setting.

Valid Range:

VI_OFF - Manual

VI_ON - Default (Default Value)

Note : Field "Ratio" is only valid when "Manual" is selected.

ratioRBWto1Span

Variable Type       ViReal64

Ratio between the Resolution Bandwidth and the Span.

Valid range : 0.001 to 0.1

Default value : 0.017

Note : This ratio is used RBW->AUTO is selected.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Control) out of range.

BFFC0003  Parameter 3 (Ratio RBW to 1 Span) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_RBW_VBW

ViStatus AVPA_Set_RBW_VBW (ViSession instrumentHandle, ViBoolean typeofControl,

ViReal64 ratioVBWto1RBW);

Purpose

This function is used to set up the ratio between VBW and RBW. This ratio

is used to estimate the appropriate setting for VBW when VBW-> AUTO is

selected. (cf. function "Set VBW").

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofControl

Variable Type       ViBoolean

Enables manual setting of the ration VBW to RBW. Resets the VBW / RBW

coupling to its default setting.

Data Range:

VI_OFF - Manual

VI_ON - Default (Default Value)

Note : Field "Ratio" is only valid when "Manual" is selected.

ratioVBWto1RBW

Variable Type       ViReal64

Ratio between the Video Bandwidth and the Resolution Bandwidth.

Valid Range:

1,3 step from 3 to 0.003 (3, 1, 0.3, 0.1, ...)

Default value : 1

Note : This ratio is used when VBW -> AUTO is selected.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Control) out of range.

BFFC0003  Parameter 3 (Ratio VBW to 1 RBW) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_RefLevel

ViStatus AVPA_Set_RefLevel (ViSession instrumentHandle, ViReal64 referenceLevel);

Purpose

This function is used to set up the Reference Level value but not the

reference unit.

Note : the reference unit can be chosen by using the function "set

reference unit".

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceLevel

Variable Type       ViReal64

Set up the Reference Level.

Data Range: -105 dBm to 60 dBm

Default Value : 0 with the current unit.

Note : The unit can be set in the function "Set Reference Unit".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Reference Level) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_RefOffset

ViStatus AVPA_Set_RefOffset (ViSession instrumentHandle,

ViBoolean refLevelOffsetActivation,

ViReal64 referenceLevelOffsetdB);

Purpose

This function is used to set the reference level Offset and to activate

or desactivate it.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

refLevelOffsetActivation

Variable Type       ViBoolean

This binary switch allows to activate or desactivate the reference

level offset.

Valid Range :

VI_ON : Active

VI_OFF : Inactive (Default)

referenceLevelOffsetdB

Variable Type       ViReal64

Value of the Reference Level Offset.

Valid Range : 0 to  +/- 100 dB

Default Value: 0 dB

Note : The unit is always the dB

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (RefLevel Offset Activation) out of range.

BFFC0003  Parameter 3 (RefLevel Offset) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_RefUnit

ViStatus AVPA_Set_RefUnit (ViSession instrumentHandle, ViInt16 referenceUnit);

Purpose

This function is used to set up the reference unit. The unit remains

valid until it changed using this same function.

It is thus unnecessary to set up the unit each time the reference level

changes.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceUnit

Variable Type       ViInt16

Selects the unit of the reference level.

Valid Range :

0 - dBm (Default)

1 - dBmV

2 - dBV

3 - dBVemf

4 - dBpW

6 - Volts

7 - Watts

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Reference Unit) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_Scale

ViStatus AVPA_Set_Scale (ViSession instrumentHandle, ViInt16 scale);

Purpose

This function is used to set up the scale of the Y axis.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

scale

Variable Type       ViInt16

Selects the scale of the Y axis. Either described in dB per screen

division or linearly.

Valid Range:

dB / Div

0 - 10 dB (Default)

1 - 5 dB

2 - 2 dB

3 - 1 dB

4 - 0.5 dB

Linear

5 - x 1

6 - x 2

7 - x 5

8 - x 10

Notes :

(1) When the scale, previously logarithmic, becomes linear the

reference unit changes from dBm to Volts.

(2) When the scale, previously linear, becomes logarithmic the

reference unit changes from Volts to dBm.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Scale) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_Span

ViStatus AVPA_Set_Span (ViSession instrumentHandle, ViInt16 spanType,

ViReal64 spanValueHz);

Purpose

This function lets the user specify the frequency span.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

spanType

Variable Type       ViInt16

The Slide "Type of Control" lets the user choose between : Manual,

Full Span and Zero Span.

Valid Range :

0 - Manual (Default)

1 - Full Span

2 - Zero Span

Notes :

(1) The other parameter of this function is "Span Value" which is

only valid when "0-Manual" is selected by this control.

(2) Full Span : Center Frequency = middle of the frequency range and

Span = frequency range.

(3) Zero Span : Span = 0 Hz, Time Domain analysis.

spanValueHz

Variable Type       ViReal64

This control is used to set up the value of the span when the "Type

of Control" is on "Manual".

Valid Range :

0 to 3 GHz for the R3263 and R3463

0 to 8 GHz for the R3465

0 to 26.5 GHz for the R3272

Default Value: 100 MHz

Note : The frequency span is always expressed in Hz.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Span Type) out of range.

BFFC0003  Parameter 3 (Span Value) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_SweepMode

ViStatus AVPA_Set_SweepMode (ViSession instrumentHandle, ViBoolean sweepMode,

ViBoolean windowSweep);

Purpose

This function is used to determine whether the sweep is continuous or

single and to activate or desactivate the Window Sweep.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sweepMode

Variable Type       ViBoolean

This binary switch lets the user choose between continuous or single

sweep.

Valid Range:

VI_OFF - Single Sweep

VI_ON - Continuous Sweep (Default)

Notes :

(1) Continuous Sweep: a new sweep is started whenever a sweep ended.

Sweep starts now.

(2) Single Sweep: After one sweep, the sweep stops.

windowSweep

Variable Type       ViBoolean

This binary switch lets the user choose whether the sweep is

performed only within the Measurement Window or not.

Valid Range:

VI_OFF - OFF (Default)

VI_ON - ON

Note : Set up the window using the function "Set Measurement Window".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Sweep Mode) out of range.

BFFC0003  Parameter 3 (Window Sweep) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_SweepTime

ViStatus AVPA_Set_SweepTime (ViSession instrumentHandle,

ViBoolean typeofControl, ViReal64 sweepTimesec);

Purpose

This function is used to set up the Sweep Time.

Automatic sweep time selection can be specified, in which case a sweep

time is chosen, by the analyzer, depending on the current setting of RBW,

VBW and SPAN.

SPAN/(RBW * Min(RBW,VBW) * 0.5) = SWEEP TIME. (AUTO mode)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofControl

Variable Type       ViBoolean

The Type of Control lets the user choose betwen a manual control or

an automatic control of the sweep time.

Valid Range:

VI_OFF - Manual

VI_ON - AUTO (Default)

Note : Automatic control means that the analyzer itself will

determine the best sweep time depending on the span, RBW and VBW

settings.

sweepTimesec

Variable Type       ViReal64

Value of the sweep time.

Valid Range : 0.05 (Default) to 1000 sec

Note :

(1) This parameter has no importance if AUTO is selected.

(2) The sweep Time is always expressed in second

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Control) out of range.

BFFC0003  Parameter 3 (Sweep Time) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_TraceAMode

ViStatus AVPA_Set_TraceAMode (ViSession instrumentHandle, ViInt16 traceAMode,

ViInt16 averageCount, ViBoolean averageMode);

Purpose

This function is used to set up the TRACE A mode. Once set up the mode

becomes active immediately.

Note : if sweep mode is SINGLE, then mode activation will become visible

at the next sweep!

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

traceAMode

Variable Type       ViInt16

Selects the mode for the TRACE A.

Valid Range :

0 - Write (Default)

1 - View

2 - Blank

3 - Max Hold

4 - Min Hold

5 - Averaging.

6 - Clear

averageCount

Variable Type       ViInt16

Sets the number of sweeps over which averaging should be performed.

Data Range : 2 to 999

Default Value : 20

Note : This setting is ignored if the trace A mode is not on

"Averaging".

averageMode

Variable Type       ViBoolean

Specifies the average mode.

Valid Range :

VI_OFF - Single

VI_ON - Continuous (Default)

Note : This setting is ignored when trace A mode is not on

"Averaging".

Let the following notation be defined:

n      current averaging count

N      Averaging count specified (see control Averaging Count)

Yn     Trace data for nth average

<Yn>   Averaged data for nth average

<Yn-1> Averaged data for n-1 th average

Sigma  Sum of all the data up to the nth point.

Algorithm 1

[n<=N]  <Yn> = Sigma/n

Algorithm 2

[N<n]   <Yn> = ((N-1) - <Yn-1>) / N + <Yn> / N

* When Continuous is selected averaging is repeated using algorithm 2

even after the specified average count has been reached.

* When Single is selected the analyzer will automatically leave the

averaging mode for View mode, as soon as the specified average count

has been reached.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace A Mode) out of range.

BFFC0003  Parameter 3 (Average Count) out of range.

BFFC0004  Parameter 4 (Average Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_TraceBMode

ViStatus AVPA_Set_TraceBMode (ViSession instrumentHandle, ViInt16 traceBMode);

Purpose

This function is used to set up the mode of the Trace B. Once set up the

mode becomes active immediately.

Note : Trace B has not the same purpose of Trace A. Trace B does not

have a Write mode.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

traceBMode

Variable Type       ViInt16

Selects the mode of the Trace B.

Valid Range:

0 - B Store

1 - B View

2 - B Blank (Default)

Note : B Store means that the spectrum analyser store the Trace A

into the Trace B.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace B Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_TraceDetector

ViStatus AVPA_Set_TraceDetector (ViSession instrumentHandle,

ViInt16 detectorMode);

Purpose

This function is used to select the Trace Detector Mode that will be used

to display the data.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

detectorMode

Variable Type       ViInt16

This control determines the detector mode that will now be used.

Valid Range:

0 - Posi-Nega (Default)

1 - Positive

2 - Negative

3 - Sample

Notes :

(1) Posi-Nega:this is the normal mode.

(2) Positive:the maximum level measured since the last point is

assigned to the screen point

(3) Negative:the minimum level measured since the last point is

assigned to the screen point

(4) Sample:the level at the screen point is taken for the screen

point.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Detector Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_TracePoints

ViStatus AVPA_Set_TracePoints (ViSession instrumentHandle,

ViBoolean numberofPointsinTrace);

Purpose

This function is used to specify how many points the screen trace should

contain.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberofPointsinTrace

Variable Type       ViBoolean

selects the number of points in the screen trace.

Valid Range:

VI_ON - 501 points

VI_OFF - 1001 points (Default Value)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Number of points in trace) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_TriggerMode

ViStatus AVPA_Set_TriggerMode (ViSession instrumentHandle,

ViInt16 triggerSource,

ViBoolean polarityofEXTERNALTrigger,

ViInt16 levelinforVideoTrigger);

Purpose

This function is used to set up the Trigger Mode, the polarity of an

external trigger and the level for the video trigger.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerSource

Variable Type       ViInt16

Selects the trigger source.

Data Range:

0 - Free run (Default)

1 - Line

2 - Video

3 - External

Notes:

(1) Free run : no trigger, repeat sweep

(2) Line : synchronization with AC supply frequency

(3) Video : Starts the sweep when level of video signal

is over the trigger level

(4) External : synchronization with a TTL signal connected

to the "external trigger input" connector

at the rear panel of the analyzer

polarityofEXTERNALTrigger

Variable Type       ViBoolean

Selects the polarity of the external trigger signal.

Valid Range:

VI_0FF : -Slope

VI_ON : +Slope (Default Value)

levelinforVideoTrigger

Variable Type       ViInt16

Value of the level of the video trigger.

Valid Range : 0 to 100 %

Default Value : 30 %

Note : this value is expressed in % of the Y axis of the display

screen.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trigger Source) out of range.

BFFC0003  Parameter 3 (Polarity of Ext. Trigger) out of range.

BFFC0004  Parameter 4 (Level for Video Trigger) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Set_VBW

ViStatus AVPA_Set_VBW (ViSession instrumentHandle, ViInt16 VBWSetting);

Purpose

This function is used to set up the bandwidth of the video filter.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

VBWSetting

Variable Type       ViInt16

This control is used to specify the value of the bandwidth of the

video filter (VBW).

Valid Range:

0 - AUTO  (Default)

1 - 1 Hz

2 - 3 Hz

3 - 10 Hz

4 - 30 Hz

5 - 100 Hz

6 - 300 Hz

7 - 1 kHz

8 - 3 kHz

9 - 10 kHz

10 - 30 kHz

11 - 100 kHz

12 - 300 kHz

13 - 1 MHz

14 - 3 MHz

15 - 5 MHz

Notes :

(1) Automatic VBW can be selected in which case the analyzer will

automatically choose VBW according to RBW.

(2) The ratio VBW:RBW can be modified using the "Set RBW/VBW

Coupling" function.

(3) For noise averaging, VBW is often chosen 1/10 under RBW (RBW =

300 kHz -> VBW = 30 kHz)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (VBW Setting) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_Take_Sweep

ViStatus AVPA_Take_Sweep (ViSession instrumentHandle, ViInt16 startSweep);

Purpose

This function is used to start the sweep in different modes.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

startSweep

Variable Type       ViInt16

Specifies the way to start the sweep.

Valid Range:

0 - Continuous Mode

1 - Single Sweep  (Default)

2 - Reset & Start Sweep

3 - Take Sweep

Note : Continuous mode can also be started by the function "Set Sweep

Mode". However, this function should be used to command the analyzer

to sweep once in single sweep mode, or to reset & start sweeping or

to execute a take sweep. In take sweep mode the analyzer will do

nothing else until sweep is ended.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Start Sweep) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVPA_WriteInstrData

ViStatus AVPA_WriteInstrData (ViSession instrumentHandle, ViString writeBuffer);

Purpose

This function writes commands and queries to the instrument to modify

parameters and query device settings.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

writeBuffer

Variable Type       ViString

The user can use this control to send common commands and queries to

the instrument. This control can also be used to write any valid

command to the instrument.

Default Value:  "*RST"

Note : The command or query to be sent to the instrument may be a

literal enclosed in double quotes i.e. "*RST" or may be contained in

a variable of type string.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.