ADVANTEST R3x61 Spectrum Analyzer (VISA)

________________________________________________________________________________

Introduction:

This instrument driver provides programming support for ADVANTEST R3x61 Spectrum Analyzer (VISA).

It contains functions for opening, configuring, taking measurements from, and

closing the instrument.

________________________________________________________________________________

Assumptions:

To successfully use this module, the following conditions must be met:

For GPIB instrument drivers:

-  the instrument is connected to the GPIB.

-  the GPIB address supplied to the initialize function must match the

GPIB address of the instrument.

For VXI instrument drivers:

-  the instrument is installed in the VXI mainframe and you are

using one of the following controller options:

Embedded controller

MXI

MXI2

GPIB-VXI

-  the logical address supplied to the initialize function must

match the logical address of the instrument.

For RS-232 instrument drivers:

-  the instrument is connected to the RS-232 interface.

-  the COM port, baud rate, parity, and timeout supplied to the

initialize function must match the settings of the instrument.

________________________________________________________________________________

Error and Status Information:

Each function in this instrument driver returns a status code that

either indicates success or describes an error or warning condition.

Your program should examine the status code from each call to an

instrument driver function to determine if an error occurred.

The general meaning of the status code is as follows:

Value                  Meaning

-------------------------------

0                      Success

Positive Values        Warnings

Negative Values        Errors

The description of each instrument driver function lists possible error

codes and their meanings

________________________________________________________________________________

How To Use This Document:

Use this document as a programming reference manual.

It describes each function in the

ADVANTEST R3x61 Spectrum Analyzer (VISA)

instrument.  The functions appear in alphabetical order, with a description

of the function and its C syntax, a description of each parameter, and a list

of possible error codes.

________________________________________________________________________________

Function Tree Layout:

ADVANTEST R3x61 Spectrum Analyzer (VISA)  Function Name:

Initialize                                  AVR3x61_init

Application Functions

Configure And Read Measurement           AVR3x61_configRead

Configuration Functions

Configure Frequency

Set Frequency                         AVR3x61_Set_Freq

Set Frequency Linear Span             AVR3x61_Set_LinearSpan

Set Frequency Log Span                AVR3x61_Set_LogSpan

Set Frequency Offset                  AVR3x61_Set_FreqOffset

Set Frequency Step Size               AVR3x61_Set_FreqStepSize

Set Frequency Counter                 AVR3x61_Set_FreqCounter

Configure Amplitude

Set Reference Level                   AVR3x61_Set_RefLevel

Set Reference Unit                    AVR3x61_Set_RefUnit

Set Scale                             AVR3x61_Set_Scale

Set Reference Level Offset            AVR3x61_Set_RefOffset

Set Input Attenuation                 AVR3x61_Set_Att

Configure Bandwidths

Set RBW                               AVR3x61_Set_RBW

Set VBW                               AVR3x61_Set_VBW

Set All Auto Couple                      AVR3x61_Set_All_Auto_Couple

Configure Sweep

Set Sweep Time                        AVR3x61_Set_SweepTime

Set Sweep Mode                        AVR3x61_Set_SweepMode

Set Trigger                           AVR3x61_Set_Trigger

Configure Trace

Set Trace A Mode                      AVR3x61_Set_TraceAMode

Set Trace B Mode                      AVR3x61_Set_TraceBMode

Set Trace Detector Mode               AVR3x61_Set_TraceDetector

Configure Display Line

Set Display Line                      AVR3x61_Set_DisplayLine

Configure Measurement Window

Set Measurement Window                AVR3x61_Set_MeasurementWindow

Configure Normal&Delta Markers

Set Normal Marker                     AVR3x61_Set_NormalMarker

Set Delta Marker                      AVR3x61_Set_DeltaMarker

Set Delta Marker Options              AVR3x61_Set_DeltaMarkerOptions

Delete All Markers                    AVR3x61_Delete_Markers

Configure Peak Search

Set Peak Search Options               AVR3x61_Set_PeakSearchOptions

Action/Status Functions

Execute Peak Search                      AVR3x61_Search_Peak

Execute Peak To                          AVR3x61_Execute_PeakTo

Marker Delta To Settings                 AVR3x61_Set_MarkerDeltaTo

Trace Action

Execute Trace Math                    AVR3x61_Execute_TraceMath

Normalizing Mode                      AVR3x61_NormalizingMode

Start/Stop Trace Average              AVR3x61_Control_TraceAverage

Configuration Read

Read Frequency Settings               AVR3x61_Read_FreqSettings

Read Frequency Offset                 AVR3x61_Read_FreqOffset

Read Frequency Log Span               AVR3x61_Read_LogSpan

Read Frequency Step Size              AVR3x61_Read_FreqStepSize

Read Frequency Counter                AVR3x61_Read_FreqCounter

Read Reference Level                  AVR3x61_Read_RefLevel

Read Reference Level Offset           AVR3x61_Read_RefLevelOffset

Read Reference Unit                   AVR3x61_Read_RefUnit

Read Scale                            AVR3x61_Read_Scale

Read Attenuation                      AVR3x61_Read_Attenuation

Read RBW&VBW                          AVR3x61_Read_RBWandVBW

Read Sweep Time                       AVR3x61_Read_SweepTime

Read Sweep Mode                       AVR3x61_Read_SweepMode

Read Trigger                          AVR3x61_Read_Trigger

Read Display Line                     AVR3x61_Read_DisplayLine

Read Measurement Window               AVR3x61_Read_MeasurementWindow

Read Peak Options                     AVR3x61_Read_Peak_Options

Status Read

Read Trace Status                     AVR3x61_Read_TraceStatus

Read Trace Detector                   AVR3x61_Read_TraceDetector

Read Marker Status                    AVR3x61_Read_MarkerStatus

Read Delta Marker Options             AVR3x61_Read_DeltaMarkerOptions

Data Functions

Trace Function

Read Trace Data                       AVR3x61_Read_TraceData

Markers Functions

Read Normal Marker                    AVR3x61_Read_NormalMarker

Utility Functions

Write To Instrument                      AVR3x61_WriteInstrData

Read Instrument Data                     AVR3x61_readInstrData

Reset                                    AVR3x61_reset

Self-Test                                AVR3x61_selfTest

Error-Query                              AVR3x61_errorQuery

Error Message                            AVR3x61_errorMessage

Revision Query                           AVR3x61_revisionQuery

Calibration                              AVR3x61_Calibration

Display Label                            AVR3x61_Display_Label

Read Status Byte                         AVR3x61_Read_StatusByte

Clear Status Byte                        AVR3x61_Clear_StatusByte

Frequency or Time Conversion

Frequency Conversion                  AVR3x61_Convert_Frequency

Time Conversion                       AVR3x61_Convert_Time

Close                                       AVR3x61_close

________________________________________________________________________________

ADVANTEST R3x61 Spectrum Analyzer (VISA)

This instrument module provides programming support for the Advantest

R3x61 Series Spectrum Analyzers : R3261A/AN/B and R3361A/AN/B. The module

is divided into the following functions:

Functions/Classes:

(1) Initialize:

This function initializes the instrument and sets it to a default

configuration.

(2) Application Functions: (Class)

This class contains high-level, test and measurement routines.  These

examples call other instrument driver functions to configure, start, and

read from the instrument.

(3) Configuration Functions: (Class)

This class of functions configures the instrument by setting acquisition

and system configuration parameters.

(4) Action/Status Functions: (Class)

This class of functions begins or terminates an acquisition. It also

provides functions which allow the user to determine the current status

of the instrument.

(5) Data Functions: (Class)

This class of functions transfers data to or from the instrument.

(6) Utility Functions: (Class)

This class of functions provides lower level functions to communicate

with the instrument, change instrument parameters and work with antenna.

(7) Close:

This function takes the instrument offline.

________________________________________________________________________________

The following functions are in alphabetical order.

________________________________________________________________________________

AVR3x61_Calibration

ViStatus AVR3x61_Calibration (ViSession instrumentHandle,

ViInt16 elementtoCalibrate,

ViReal64 calibrationLevel,

ViBoolean calibrationCorrection,

ViBoolean frequencyCorrection);

Purpose

This function specifies the type of calibration required and starts it.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

elementtoCalibrate

Variable Type       ViInt16

specifies which element to calibrate

Valid Range :

0 - All (Default)

1 - Total Gain

2 - Input Attenuator

3 - IF step Amplifier

4 - RBW switch

5 - Logarithmic linearity

6 - Amplifier Magnitude

7 - TG tracking

calibrationLevel

Variable Type       ViReal64

specifies the level of the output signal of calibration.

Valid Range: -20 dB to -30 dB (Default)

calibrationCorrection

Variable Type       ViBoolean

specifies whether to activate or deactivate the calibration factor

obtained after the calibration.

Valid Range:

VI_ON  (1) - ON

VI_OFF (0) - OFF

frequencyCorrection

Variable Type       ViBoolean

specifies whether to activate or deactivate the frequency correction.

Valid Range:

VI_ON  (1) - ON (Default)

VI_OFF (0) - OFF

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Element to Calibrate) out of range.

BFFC0003  Parameter 3 (Calibration Level) out of range.

BFFC0004  Parameter 4 (Calibration Correction) out of range.

BFFC0005  Parameter 5 (Frequency Correction) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Clear_StatusByte

ViStatus AVR3x61_Clear_StatusByte (ViSession instrumentHandle);

Purpose

This function is used to clear the status byte register.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_close

ViStatus AVR3x61_close (ViSession instrumentHandle);

Purpose

This function performs the following operations:

viClose (instrSession) and viClose (rmSession).

Note : the instrument must be reinitialized to use it again.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_configRead

ViStatus AVR3x61_configRead (ViSession instrumentHandle, ViBoolean tracetoRead,

ViInt16 typeofSearch, ViInt16 traceDataArray[],

ViReal64 *sweepTime, ViChar sweepUnit[],

ViReal64 *markerPosition, ViReal64 *markerLevel);

Purpose

This function performs the following operations:

1 - returns the trace Data of the chosen trace in an array.

2 - reads the sweep time : value and unit.

3 - searches the desired peak and returns its coordinates.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

tracetoRead

Variable Type       ViBoolean

selects the trace to store in an array.

Valid Range :

VI_ON  (1) - Trace A (Default)

VI_OFF (0) - Trace B

typeofSearch

Variable Type       ViInt16

This control selects the desired peak marker search.

Valid Range :

0 - Peak Search (Default)

1 - Next Peak

2 - Next Peak Left

3 - Next Peak Right

4 - Minimum Search

5 - Next Minimum

traceDataArray

Variable Type       ViInt16[]

This is the array where the function will write the trace data.

sweepTime

Variable Type       ViReal64 (passed by reference)

returns the value of the sweep time value.

sweepUnit

Variable Type       ViChar[]

returns the sweep time unit.

markerPosition

Variable Type       ViReal64 (passed by reference)

returns the position of the marker normal ie the position of the

peak searched.

markerLevel

Variable Type       ViReal64 (passed by reference)

returns the level of the marker normal ie the level of the peak

searched.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace to Read) out of range.

BFFC0003  Parameter 3 (Type of Search) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Control_TraceAverage

ViStatus AVR3x61_Control_TraceAverage (ViSession instrumentHandle,

ViBoolean trace, ViInt16 traceAverage);

Purpose

This function is used to control start, stop, pause or continue of trace

average.

Here Trace Average can be stopped, and restarted at will without

redefining the trace count or execution mode.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

trace

Variable Type       ViBoolean

selects the trace on which you want to work

Valid Range :

VI_ON  (1) - Trace A (Default)

VI_OFF (0) - Trace B

traceAverage

Variable Type       ViInt16

Specifies the action to perform in respect to Trace Averaging.

Valid Range:

0 - Start (Default)

1 - Stop

2 - Pause

3 - Continue

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace) out of range.

BFFC0003  Parameter 3 (Trace Average) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Convert_Frequency

ViStatus AVR3x61_Convert_Frequency (ViSession instrumentHandle,

ViReal64 inputFrequencyValue,

ViReal64 *outputFrequencyValue,

ViChar frequencyUnit[]);

Purpose

This function is used to convert the frequency value returned by the

spectrum analyzer X Hz to X Hz or Y kHz/MHz/GHz.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

inputFrequencyValue

Variable Type       ViReal64

Frequency value returns by the spectrum analyzer and always expressed

in Hz.

outputFrequencyValue

Variable Type       ViReal64 (passed by reference)

returns X the value of the frequency in the following expressions : X

Hz, X kHz, X MHz and X GHz.

frequencyUnit

Variable Type       ViChar[]

returns the best frequency's unit between Hz, kHz, MHz and GHz.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Convert_Time

ViStatus AVR3x61_Convert_Time (ViSession instrumentHandle,

ViReal64 inputTimeValue,

ViReal64 *outputTimeValue, ViChar timeUnit[]);

Purpose

This function is used to convert the time value returned by the spectrum

analyzer X s to X s or Y ms,Y us, Y ns.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

inputTimeValue

Variable Type       ViReal64

time value returns by the spectrum analyzer and always expressed in

seconds.

outputTimeValue

Variable Type       ViReal64 (passed by reference)

returns X the value of the time in the following expressions : X s, X

ms, X us or X ns.

timeUnit

Variable Type       ViChar[]

returns the best time's unit between s, ms, us, and ns.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Delete_Markers

ViStatus AVR3x61_Delete_Markers (ViSession instrumentHandle);

Purpose

This function is used to deactivate all the markers that are active.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Display_Label

ViStatus AVR3x61_Display_Label (ViSession instrumentHandle, ViBoolean showLabel,

ViString label);

Purpose

This function is used to display a label on the screen or to delete it.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

showLabel

Variable Type       ViBoolean

This binary switch is used to display the specified label or to

delete the current label.

Valid Range:

VI_ON  (1) - Show

VI_OFF (0) - Delete (Default)

Note : If the current label is deleted, the label is lost and a new

label will have to be specified the next time a label must be shown.

label

Variable Type       ViString

Specifies the label to display on the upper left of the screen.

Data Range : A character string of at most 49 characters.

Note : This string is ignored if "Show Label" is set to "delete".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Show Label) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_errorMessage

ViStatus AVR3x61_errorMessage (ViSession instrumentHandle, ViStatus statusCode,

ViChar message[]);

Purpose

This function takes the Status Code returned by the instrument driver

functions, interprets it and returns it as a user readable string.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  VI_NULL

statusCode

Variable Type       ViStatus

This control accepts the Status Code returned from the instrument

driver functions.

Default Value:

0 - VI_SUCCESS

message

Variable Type       ViChar[]

This control returns the interpreted Status Code as a user readable

message string.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFF0085  VI_WARN_UNKNOWN_STATUS

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_errorQuery

ViStatus AVR3x61_errorQuery (ViSession instrumentHandle, ViInt32 *errorCode,

ViChar errorMessage[]);

Purpose

This function reads an error code from the instrument's error queue.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

errorCode

Variable Type       ViInt32 (passed by reference)

This control returns the error code read from the instrument's error

queue.

errorMessage

Variable Type       ViChar[]

This control returns the error message string read from the

instrument's error message queue.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Execute_PeakTo

ViStatus AVR3x61_Execute_PeakTo (ViSession instrumentHandle, ViBoolean peakTo);

Purpose

This function is used to assign the peak value to one of the following

parameters : Center Frequency or Reference Level.

If Peak To -> Center Frequency, the frequency of the highest peak becomes

the center frequency.

If Peak To -> Reference Level, the level of the highest peak becomes the

reference level.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

peakTo

Variable Type       ViBoolean

selects the element which will be modified with the peak value.

Data range :

VI_ON  (1) - Center Frequency (Default)

VI_OFF (0) - Reference Level

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Peak To) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Execute_TraceMath

ViStatus AVR3x61_Execute_TraceMath (ViSession instrumentHandle,

ViInt16 traceMath);

Purpose

This function allows to work with the traces A and B.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

traceMath

Variable Type       ViInt16

Specifies the kind of trace math function that should be activated.

Valid Range:

0 - None

1 - A <--> B

2 - A - B --> A

3 - B - A --> A

4 - A - DL --> A (DL = Display Line)

5 - B - DL --> B

Default Value:

0 - None

Note : The selection "None" means that trace math is cancelled.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace Math) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_init

ViStatus AVR3x61_init (ViRsrc resourceName, ViBoolean IDQuery,

ViBoolean resetDevice, ViSession *instrumentHandle);

Purpose

This function performs the following initialization actions:

- Opens a session to the Default Resource Manager resource and a session

to the specified device using the interface and address specified in the

Resource_Name control.

- Performs an identification query on the Instrument.

- Resets the instrument to a known state.

- Sends initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

- Returns an Instrument Handle which is used to differentiate between

different sessions of this instrument driver.

- Each time this function is invoked a Unique Session is opened.  It is

possible to have more than one session open for the same resource.

Parameter List

resourceName

Variable Type       ViRsrc

This control specifies the interface and address of the device that

is to be initialized (Instrument Descriptor). The exact grammar to be

used in this control is shown in the note below.

Default Value:  "GPIB::1"

Notes:

(1) Based on the Instrument Descriptor, this operation establishes a

communication session with a device.  The grammar for the Instrument

Descriptor is shown below.  Optional parameters are shown in square

brackets ([]).

(2) Interface   Grammar

------------------------------------------------------

GPIB        GPIB[board]::primary address[::secondary address]

[::INSTR]

ASRL        ASRL[board][::INSTR]

The GPIB and ASRL keywords are used respectively with GPIB and serial

instruments.

The default value for optional parameters are shown below.

Optional Parameter          Default Value

-----------------------------------------

board                       0

secondary address           none - 31

(3) the serial interface is initialized with 8 databits, 1 stopbit,

no parity, baud rate = 9600, flow control = XonXoff and timeout =

10000 ms

IDQuery

Variable Type       ViBoolean

This control specifies if an ID Query is sent to the instrument

during the initialization procedure.

Valid Range:

VI_OFF (0) - Skip Query

VI_ON  (1) - Do Query (Default Value)

Notes:

(1) Under normal circumstances the ID Query ensures that the

instrument initialized is the type supported by this driver. However

circumstances may arise where it is undesirable to send an ID Query

to the instrument.  In those cases; set this control to "Skip Query"

and this function will initialize the selected interface, without

doing an ID Query.

(2) If the identification is skipped, the driver considers that the

spectrum analyzer used is an R3261A

resetDevice

Variable Type       ViBoolean

This control specifies if the instrument is to be reset to its

power-on settings during the initialization procedure.

Valid Range:

VI_OFF (0) - Don't Reset

VI_ON  (1) - Reset Device (Default Value)

Note : If you do not want the instrument reset. Set this control to

"Don't Reset" while initializing the instrument.

instrumentHandle

Variable Type       ViSession (passed by reference)

This control returns an Instrument Handle that is used in all

subsequent function calls to differentiate between different sessions

of this instrument driver.

Notes:

(1) Each time this function is invoked a Unique Session is opened.

It is possible to have more than one session open for the same

resource.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (ID Query) out of range.

BFFC0003  Parameter 3 (Reset Device) out of range.

BFFC0011  ID Query Failed VI_ERROR_FAIL_ID_QUERY

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_NormalizingMode

ViStatus AVR3x61_NormalizingMode (ViSession instrumentHandle, ViBoolean trace,

ViInt16 normalizeMode);

Purpose

This function makes waveform comparison easily by treating reference

signal as a display line data.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

trace

Variable Type       ViBoolean

selects the the trace which you want to work on

Valid Range :

VI_ON  (1) - Trace A (Default)

VI_OFF (0) - Trace B

normalizeMode

Variable Type       ViInt16

This slide is used to select the point of the normalizing mode that

you want to execute.

Valid Range :

0 - Normalize ON

1 - Save Corr Dat (Default)

2 - Instant Normalize

3 - Normalize OFF

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace) out of range.

BFFC0003  Parameter 3 (Normalize Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_readInstrData

ViStatus AVR3x61_readInstrData (ViSession instrumentHandle,

ViInt32 numberBytesToRead, ViChar readBuffer[],

ViInt32 *numBytesRead);

Purpose

This function reads data from the instrument's output buffer and returns

it to the specified variable in memory.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) If valid data is not available at the instrument's output buffer when

this function is called the instrument will hang up and the function will

not return until it times out.  If the time-out is disabled this function

will hang indefinitely and it may be necessary to reboot the computer to

break out.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

numberBytesToRead

Variable Type       ViInt32

The number of bytes specified by this control should be greater than

or equal to the number of bytes which are to be read from the

instrument. If the actual number of bytes to be read is greater than

the number this control specifies then multiple reads will be

required to empty the instrument's output buffer.

If the instrument's output buffer is not emptied the instrument may

return invalid data when future reads are performed.

Default Value:  50 (See NOTE 2)

Notes:

(1) If NO DATA is available at the instrument's output buffer when

this function is called the instrument will hang up and the function

will not return until it times out.  If the time-out is disabled,

this function will hang indefinitely and it may be necessary to

reboot the computer.

(2) If the number of bytes expected is greater than 50 the value of

this control may be increased. If the actual number of bytes read is

larger than the declared size of the read buffer a run-time error

will be generated.

readBuffer

Variable Type       ViChar[]

The incoming data from the instrument is placed into this variable.

Notes:

(1) Because the instrument may return both numeric and text data in

response to queries this function returns the data in string format.

(2) This function does not overwrite any old data left in the string

variable from the last time the function was called. String data in

LabWindows/CVI is terminated with an ASCII null(0x0) and string

manipulation functions will only recognize data before the ASCII

null.

(3) The declared size of the string variable must be greater than the

actual number of bytes read from the instrument if it is not a

run-time error will be generated.

numBytesRead

Variable Type       ViInt32 (passed by reference)

This variable contains the actual number of bytes read from the

instrument. This is the value which is returned by the read function.

Notes:

(1) If the actual number of bytes read is less than the number of

bytes specified in the Number Bytes To Read control then the output

buffer has probably been emptied. If the read function fails and the

number of bytes read is 0, the most probable cause for the failure is

there was no data available at the instrument's output buffer.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_Attenuation

ViStatus AVR3x61_Read_Attenuation (ViSession instrumentHandle,

ViReal64 *valueindB, ViInt16 *typeofControl);

Purpose

This function reads the value of the input attenuation and returns

whether they are controlled by the user or by the spectrum analyzer.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

valueindB

Variable Type       ViReal64 (passed by reference)

returns the value always expressed in dB of the Input Attenuation.

typeofControl

Variable Type       ViInt16 (passed by reference)

returns whether the Input Attenuation is set up by the user or by the

spectrum analyzer.

Valid Range :

0 - by the user

1 - by the spectrum analyzer

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_DeltaMarkerOptions

ViStatus AVR3x61_Read_DeltaMarkerOptions (ViSession instrumentHandle,

ViInt16 *fixedMarkerStatus,

ViChar fixedMarkerMessage[]);

Purpose

This function allows to know whether the option the delta marker is fixed

or not.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

fixedMarkerStatus

Variable Type       ViInt16 (passed by reference)

receives the fixed delta marker status :

Valid Range :

0 - inactive

1 - active

fixedMarkerMessage

Variable Type       ViChar[]

returns the message corresponding to the Fixed Marker status.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_DisplayLine

ViStatus AVR3x61_Read_DisplayLine (ViSession instrumentHandle,

ViReal64 *levelValue,

ViInt16 *displayLineStatus,

ViChar displayLineMessage[]);

Purpose

This function returns the position and the status of the display line.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

levelValue

Variable Type       ViReal64 (passed by reference)

Returns the current or the last value (if Display Line is inactive)

of the Display Line level.

Note : the unit of the Display Line level is the reference unit.

displayLineStatus

Variable Type       ViInt16 (passed by reference)

receives the Display Line status.

Valid Range :

0 - inactive

1 - active

displayLineMessage

Variable Type       ViChar[]

receives the message corresponding to the status.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_FreqCounter

ViStatus AVR3x61_Read_FreqCounter (ViSession instrumentHandle,

ViReal64 *counterFrequencyValueHz,

ViInt16 *frequencyCounterStatus);

Purpose

This function reads the counter frequency value and its status.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

counterFrequencyValueHz

Variable Type       ViReal64 (passed by reference)

If the counter is Active, you can read its frequency value expressed

in Hz. Otherwise, you read the frequency of the marker normal on

which the counter is placed.

Note : the frequency displayed is truncated but if you want to see

the value really returned by the driver double-click on this control.

frequencyCounterStatus

Variable Type       ViInt16 (passed by reference)

receives the Frequency Counter status.

Valid Range :

0 - Inactive

1 - Active

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_FreqOffset

ViStatus AVR3x61_Read_FreqOffset (ViSession instrumentHandle,

ViInt16 *frequencyOffsetStatus,

ViReal64 *frequencyOffsetHz);

Purpose

This function is used to read the Frequency Offset expressed in Hz and

its status (active or inactive).

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyOffsetStatus

Variable Type       ViInt16 (passed by reference)

returns the status of the Frequency Offset.

Valid Range :

0 - inactive

1 - active

frequencyOffsetHz

Variable Type       ViReal64 (passed by reference)

Returns the Frequency Offset expressed in Hz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_FreqSettings

ViStatus AVR3x61_Read_FreqSettings (ViSession instrumentHandle,

ViInt16 frequencyParameter,

ViReal64 *frequencyValueHz);

Purpose

This function is used to read the currently set value of one of these

frequency parameters : center frequency, frequency span, start frequency

or stop frequency.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyParameter

Variable Type       ViInt16

Selects the frequency parameter whose setting must be queried.

Valid Range :

0 - Center Frequency (Default)

1 - Frequency Span

2 - Start Frequency

3 - Stop Frequency

frequencyValueHz

Variable Type       ViReal64 (passed by reference)

Returns the current setting value of the selected frequency parameter

expressed in Hz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Frequency Parameter) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_FreqStepSize

ViStatus AVR3x61_Read_FreqStepSize (ViSession instrumentHandle,

ViReal64 *frequencyStepSizeHz,

ViInt16 *controlStatus);

Purpose

This function is used to read the Frequency Step Size expressed in Hz and

the status of the Frequency Step Size Control (Automatic or Manual)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyStepSizeHz

Variable Type       ViReal64 (passed by reference)

Returns the Frequency Step Size expressed in Hz

controlStatus

Variable Type       ViInt16 (passed by reference)

returns the status of the Frequency Step Size control.

Valid Range :

0 - Manual

1 - Automatic

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_LogSpan

ViStatus AVR3x61_Read_LogSpan (ViSession instrumentHandle, ViReal64 *logStartHz,

ViReal64 *logStopHz);

Purpose

This function is used to read the frequency Log Start and the frequency

Log Stop values expressed in Hz.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

logStartHz

Variable Type       ViReal64 (passed by reference)

Returns the frequency Log Start value expressed in Hz

logStopHz

Variable Type       ViReal64 (passed by reference)

Returns the frequency Log Start value expressed in Hz.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_MarkerStatus

ViStatus AVR3x61_Read_MarkerStatus (ViSession instrumentHandle,

ViInt16 *markerStatus,

ViChar markerStatusMessage[]);

Purpose

This function is used to know whether the normal and delta markers are

active or not.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

markerStatus

Variable Type       ViInt16 (passed by reference)

receives the marker status.

Valid Range :

0 - Markers Normal and Delta inactive

1 - Only Marker Normal active

2 - Markers Normal and Delta active

markerStatusMessage

Variable Type       ViChar[]

returns the marker status message corresponding to the marker status

number : "Markers inactive", "Only Normal Marker active" or "Normal

and Delta markers active".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_MeasurementWindow

ViStatus AVR3x61_Read_MeasurementWindow (ViSession instrumentHandle,

ViInt16 *windowStatus,

ViChar windowStatusMessage[],

ViReal64 *xaxisPosition,

ViReal64 *xaxisWidth,

ViReal64 *yaxisPosition,

ViReal64 *yaxisWidth);

Purpose

This function is used to read the features of the measurement window and

to know whether the Measurement window is active or not.

Note : this function can be used only if you are working with a non nul

span.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

windowStatus

Variable Type       ViInt16 (passed by reference)

receives the status of the measurement window.

Valid Range :

0 - inactive

1 - active

windowStatusMessage

Variable Type       ViChar[]

returns the message according to the status number

xaxisPosition

Variable Type       ViReal64 (passed by reference)

current or last position (if window is inactive) of the Measurement

Window expressed in Hz.

Notes :

(1) this function can be used only if you are working with a non nul

span.

(2) window position = position of the window's center and unit is Hz.

xaxisWidth

Variable Type       ViReal64 (passed by reference)

current or last width (if window is inactive) of the Measurement

Window expressed in Hz.

Notes:

(1) this function can be used only if you are working with a non nul

span.

(2) window position = position of the window's center and unit is Hz.

yaxisPosition

Variable Type       ViReal64 (passed by reference)

This function returns the current or last (if window is inactive)

center's position of the measurement window along the Y-axis. The

unit is the Reference Unit.

yaxisWidth

Variable Type       ViReal64 (passed by reference)

This function returns the current or last (if window is inactive)

width's position of the measurement window along the Y-axis. The unit

is the Reference Unit.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_NormalMarker

ViStatus AVR3x61_Read_NormalMarker (ViSession instrumentHandle,

ViReal64 *normalMarkerPosition,

ViReal64 *normalMarkerLevel);

Purpose

This function returns the position and the level of the Normal Marker.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

normalMarkerPosition

Variable Type       ViReal64 (passed by reference)

returns the position of the normal marker expressed in Hz when span

is not null and in second when span is null.

normalMarkerLevel

Variable Type       ViReal64 (passed by reference)

returns the level of the normal marker without unit.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_Peak_Options

ViStatus AVR3x61_Read_Peak_Options (ViSession instrumentHandle,

ViInt16 *continuousPeakStatus,

ViChar continuousPeakMessage[],

ViInt16 *signalTrackStatus,

ViChar signalTrackMessage[]);

Purpose

This function is used to read the continuous peak status and the signal

track status.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

continuousPeakStatus

Variable Type       ViInt16 (passed by reference)

receives the continuous peak status.

Valid Range :

0 - OFF

1 - ON

continuousPeakMessage

Variable Type       ViChar[]

receives the message corresponding to the continuous peak status.

signalTrackStatus

Variable Type       ViInt16 (passed by reference)

receives the signal track status.

Valid Range :

0 - OFF

1 - ON

signalTrackMessage

Variable Type       ViChar[]

receives the message corresponding to the signal track status.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_RBWandVBW

ViStatus AVR3x61_Read_RBWandVBW (ViSession instrumentHandle,

ViBoolean bandwidthtoRead,

ViReal64 *bandwidthValueHz,

ViInt16 *typeofControl);

Purpose

This function returns the values of the resolution and video bandwidths.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

bandwidthtoRead

Variable Type       ViBoolean

selects the bandwidth that you want to read.

Valid Range :

VI_ON (1) - RBW (Default)

VI_OFF (0) - VBW

bandwidthValueHz

Variable Type       ViReal64 (passed by reference)

returns the value of the Resolution Bandwidth or of the Video

Bandwidth always expressed in Hz

typeofControl

Variable Type       ViInt16 (passed by reference)

returns whether the RBW or the VBW is set up by the user or by the

spectrum analyzer.

Valid Range :

0 - by the user

1 - by the spectrum analyzer

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Bandwidth to Read) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_RefLevel

ViStatus AVR3x61_Read_RefLevel (ViSession instrumentHandle,

ViReal64 *referenceLevelValue);

Purpose

This function is used to read the value (without unit) of the  Reference

Level.

Note : the unit is returned by the function "Read Reference Unit".

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceLevelValue

Variable Type       ViReal64 (passed by reference)

Returns the current setting value of the Reference Level.

Note : the unit is returned by the function "Read Reference Unit".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_RefLevelOffset

ViStatus AVR3x61_Read_RefLevelOffset (ViSession instrumentHandle,

ViReal64 *referenceLevelOffsetValue,

ViInt16 *typeofControl);

Purpose

This function is used to read the value (the unit is always dB) and the

status of the  Reference Level offset.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceLevelOffsetValue

Variable Type       ViReal64 (passed by reference)

Returns the current setting value of the Reference Level Offset (unit

is always dB).

typeofControl

Variable Type       ViInt16 (passed by reference)

This indicator is used to know whether the reference level offset is

active or not.

Valid Range :

0 - inactive

1 - active

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_RefUnit

ViStatus AVR3x61_Read_RefUnit (ViSession instrumentHandle,

ViInt16 *referenceUnitNumber,

ViChar referenceUnitName[]);

Purpose

This function returns the name and the number corresponding to the name

of the Reference Unit.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceUnitNumber

Variable Type       ViInt16 (passed by reference)

Returns the number corresponding to the Reference Unit

Valid Range :

0 - dBm

1 - dBmV

2 - dBuV

3 - dBuVemf

4 - dBpW

5 - dB

6 - V

referenceUnitName

Variable Type       ViChar[]

Returns the name of the Reference Unit : dBm, dBmV, dBuV, dBuVemf,

dBpW, dB or V

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_Scale

ViStatus AVR3x61_Read_Scale (ViSession instrumentHandle, ViInt16 *scaleStatus,

ViChar nameoftheScale[]);

Purpose

This function returns the name of the scale and its status.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

scaleStatus

Variable Type       ViInt16 (passed by reference)

receives the scale status :

if log mode :

0 - 10 dB/

1 - 5 dB/

2 - 2 dB/

3 - 1 dB/

4 - 0.5 dB/

5 - 0.2 dB/

6 - 0.1 dB/

if linear mode :

7 - Lin x1

8 - Lin x1.6

9 - Lin x4

10 - Lin x8

nameoftheScale

Variable Type       ViChar[]

returns the name of the scale : 10 dB/, 5 dB/, 2 dB/, 1 dB/, 0.5 dB/,

0.2 dB/, 0.1 dB/, Lin x1, Lin x1.6, Lin x4 or Lin x8

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_StatusByte

ViStatus AVR3x61_Read_StatusByte (ViSession instrumentHandle,

ViInt16 *statusByteRead);

Purpose

This function is used to read the status byte register.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

statusByteRead

Variable Type       ViInt16 (passed by reference)

returns the Status Byte.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_SweepMode

ViStatus AVR3x61_Read_SweepMode (ViSession instrumentHandle,

ViInt16 *sweepModeStatus, ViChar sweepMode[]);

Purpose

This function returns the Sweep Mode and says whether the sweep is

performed in the full screen or only in the Measurement Window.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sweepModeStatus

Variable Type       ViInt16 (passed by reference)

returns the Sweep Mode status.

Valid Range :

0 - Normal & full

20 - Single & full

1 - Normal & window

21 - Single & window

Notes :

(1) Normal - Continuous sweep

(2) Single - single sweep

(3) Full - measurement window inactive

(4) Window - measurement window active

sweepMode

Variable Type       ViChar[]

returns the Sweep Mode and says whether the sweep is performed in the

full screen or only in the Measurement Window.

Valid Range :

Normal & full

Single & full

Normal & window

Single & window

Notes :

(1) Normal - Continuous sweep

(2) Single - single sweep

(3) Full - measurement window inactive

(4) Window - measurement window active

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_SweepTime

ViStatus AVR3x61_Read_SweepTime (ViSession instrumentHandle,

ViReal64 *timeValuesecond,

ViInt16 *typeofControl);

Purpose

This function is used to read the Sweep Time value expressed always in

seconds and the status of the sweep time control (manual or automatic)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

timeValuesecond

Variable Type       ViReal64 (passed by reference)

Returns the current setting value of the Sweep Time expressed always

in second.

typeofControl

Variable Type       ViInt16 (passed by reference)

returns whether the Sweep Time is set up by the user or by the

spectrum analyzer.

Valid Range :

0 - by the user

1 - by the spectrum analyzer

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_TraceData

ViStatus AVR3x61_Read_TraceData (ViSession instrumentHandle,

ViBoolean tracetoRead, ViBoolean dataFormat,

ViInt16 traceDataArray[]);

Purpose

This function is used to read a trace. The result is send back in the

trace Data Array.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

tracetoRead

Variable Type       ViBoolean

selects the trace to store in an array.

Valid Range :

VI_ON  (1) - Trace A (Default)

VI_OFF (0) - Trace B

dataFormat

Variable Type       ViBoolean

Selects the transfer format.

Valid Range :

VI_ON  (1) - Binary (Default)

VI_OFF (0) - ASCII

traceDataArray

Variable Type       ViInt16[]

This is the array where the function will write the trace data.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace to Read) out of range.

BFFC0003  Parameter 3 (Data Format) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_TraceDetector

ViStatus AVR3x61_Read_TraceDetector (ViSession instrumentHandle,

ViInt16 *detectorModeStatus,

ViChar detectorMode[]);

Purpose

This function is used to read the current Trace Detector mode.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

detectorModeStatus

Variable Type       ViInt16 (passed by reference)

receives the Detector Mode status.

Valid Range :

0 - Normal (Posi-Nega)

1 - Positive

2 - Negative

3 - Sample

detectorMode

Variable Type       ViChar[]

returns the Detector Mode : Normal (Posi-Nega), Positive, Negative or

Sample.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_TraceStatus

ViStatus AVR3x61_Read_TraceStatus (ViSession instrumentHandle,

ViBoolean traceModetoRead,

ViInt16 *traceModeStatus,

ViChar generalStatusText[]);

Purpose

This function returns the trace mode of the Trace A and The Trace B

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

traceModetoRead

Variable Type       ViBoolean

selects the name of the trace to scan

Valid Range :

VI_ON  (1) - Trace A (Default)

VI_OFF (0) - Trace B

traceModeStatus

Variable Type       ViInt16 (passed by reference)

receives the trace mode status

Valid Range :

TRACE A :

---------

0  -  write

1  -  view

2  -  blank

3  -  norm

4  -  A-DL->A

5  -  A-B->A

6  -  B-A->A

7  -  max hold

8  -  averaging

TRACE B :

---------

0  -  write

1  -  view

2  -  blank

3  -  norm

4  -  B-DL->B

7  -  max hold

8  -  averaging

generalStatusText

Variable Type       ViChar[]

returns the mode corresponding to the status displayed in the other

output variable.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace Mode to Read) out of range.

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Read_Trigger

ViStatus AVR3x61_Read_Trigger (ViSession instrumentHandle,

ViInt16 *triggerStatus, ViChar triggerName[]);

Purpose

This function returns the trigger used.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerStatus

Variable Type       ViInt16 (passed by reference)

returns the Trigger Mode status.

Valid Range :

0 - Free Run

1 - Line

2 - Video

3 - TV_V

4 - External

5 - Single

triggerName

Variable Type       ViChar[]

returns the Trigger Name : Free Run, Line, Video, TV_V, External or

Single.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0803  Invalid Response VI_ERROR_INSTR_INTERPRETING_RESPONSE

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_reset

ViStatus AVR3x61_reset (ViSession instrumentHandle);

Purpose

This function resets the instrument to a known state and sends

initialization commands to the instrument that set any necessary

programmatic variables such as Headers Off, Short Command form, and Data

Transfer Binary to the state necessary for the operation of the

instrument driver.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_revisionQuery

ViStatus AVR3x61_revisionQuery (ViSession instrumentHandle,

ViChar instrumentDriverRevision[],

ViChar firmwareRevision[]);

Purpose

This function returns the revision numbers of the instrument driver and

instrument firmware, and tells the user with which  instrument firmware

this revision of the driver is compatible.

For example :

This instrument driver's Revision Number is "Rev 2.0, 04/99, CVI 5.0.1"

and is compatible with Instrument Firmware Revision H03.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

instrumentDriverRevision

Variable Type       ViChar[]

This control returns the Instrument Driver Software Revision.

firmwareRevision

Variable Type       ViChar[]

This control returns the Instrument Firmware Revision.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Search_Peak

ViStatus AVR3x61_Search_Peak (ViSession instrumentHandle, ViInt16 typeofSearch);

Purpose

This function performs different type of peak marker search.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofSearch

Variable Type       ViInt16

This control selects the desired peak marker search.

Valid Range :

0 - Peak Search (Default)

1 - Next Peak

2 - Next Peak Left

3 - Next Peak Right

4 - Next Peak Max/Min

5 - Minimum Search

6 - Next Minimum

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Search) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_selfTest

ViStatus AVR3x61_selfTest (ViSession instrumentHandle, ViInt16 *selfTestResult,

ViChar selfTestMessage[]);

Purpose

This function runs the instrument's self test routine and returns the

test result(s).

This instrument does not support a Self-Test, this function returns the

Warning Code 0x3FFC0103 - VI_WARN_NSUP_SELF_TEST

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

selfTestResult

Variable Type       ViInt16 (passed by reference)

This control contains the value returned from the instrument self

test.  Self-Test is not supported by this instrument this control

will always return negative one (-1).

selfTestMessage

Variable Type       ViChar[]

This control contains the string returned from the self test. This

instrument does not support a self-test and this control will always

return the message "Self-Test Not Supported".

Notes:

(1) The array must contain at least 256 elements ViChar[256].

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

3FFC0103  Self Test not supported - VI_WARN_NSUP_SELF_TEST

3FFF0005  The specified termination character was read.

3FFF0006  The specified number of bytes was read.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_All_Auto_Couple

ViStatus AVR3x61_Set_All_Auto_Couple (ViSession instrumentHandle);

Purpose

This function sets all coupled functions (RBW, VBW, and Sweep Time) to

AUTO, according to the current SPAN.

No parameter is needed.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_Att

ViStatus AVR3x61_Set_Att (ViSession instrumentHandle, ViBoolean typeofControl,

ViReal64 inputAttenuationValuedB);

Purpose

This function is used to configure the input attenuator. Normal (default)

position is AUTO. This is also the safest position.

Note : the input attenuation setting is conditioned by the minimum

attenuation setting: a value smaller than the minimum attenuation cannot

be set. This limitation is controlled by the analyzer itself.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofControl

Variable Type       ViBoolean

This Binary Switch allows to control the input attenuation manually

or to let the spectrum analyser choose its value.

Valid Range:

VI_OFF (0) - Manual

VI_ON  (1) - Automatic ( Default)

Note : the other parameter of this function is "Input Attenuation

Value" which is only valid when "VI_OFF-Manual" is selected by this

control.

inputAttenuationValuedB

Variable Type       ViReal64

Value that the input attenuation should be set to.

Data Range: 0 to 50 dB

Default Value : 10 dB

Warning: the analyzer will not set input attenuation under the

minimum attenuation setting. If a value less than the minimum is set

here the minimum attenuation will be set instead.

Note : the Input Attenuation is always expressed in dB.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Control) out of range.

BFFC0003  Parameter 3 (Input Attenuation Value) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_DeltaMarker

ViStatus AVR3x61_Set_DeltaMarker (ViSession instrumentHandle,

ViReal64 deltaPositionHzors);

Purpose

This panel is used to activate and to set the difference of

frequency/time position between the Normal Marker and the Delta  Marker.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

deltaPositionHzors

Variable Type       ViReal64

Specifies where to position the Delta Marker : this is the difference

between the frequency/time position of the Normal Marker and the

frequency/time position of the Delta Marker.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_DeltaMarkerOptions

ViStatus AVR3x61_Set_DeltaMarkerOptions (ViSession instrumentHandle,

ViBoolean deltaMarkerFixed);

Purpose

This function is used to set up the option about the Delta Marker.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

deltaMarkerFixed

Variable Type       ViBoolean

This binary switch is used to activate or deactivate the mode "Fixed

Delta Marker". This option can be used only when the span is not

null.

Valid Range :

VI_ON  (1) : Active

VI_OFF (0) : Inactive (Default)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (1/Delta Marker) out of range.

BFFC0003  Parameter 3 (Delta Marker Fixed) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_DisplayLine

ViStatus AVR3x61_Set_DisplayLine (ViSession instrumentHandle,

ViBoolean showDisplayLine,

ViReal64 leveldBxorV);

Purpose

This function is used to define the display line position and show or

hide the display line.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

showDisplayLine

Variable Type       ViBoolean

This binary switch is used to show or hide the display line.

Valid Range :

VI_ON  (1) - Show

VI_OF (0)F - Hide (Default)

leveldBxorV

Variable Type       ViReal64

Specifies the position where the display line should be shown.

Valid Range : -1000 to +1000

Note : This position is expressed in the same unit as the one defined

by the function Set Reference Unit.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Show Display Line) out of range.

BFFC0003  Parameter 3 (Position) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_Freq

ViStatus AVR3x61_Set_Freq (ViSession instrumentHandle, ViInt16 frequencyType,

ViReal64 frequencyValueHz);

Purpose

This function sets the specified frequency (Frequency Type) to the

specified value (Frequency value).

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyType

Variable Type       ViInt16

The Frequency Type determines which frequency parameter must be set

between the center frequency and the start or stop frequency.

Valid Range :

0 - Center (Default)

1 - Start

2 - Stop

frequencyValueHz

Variable Type       ViReal64

The frequency value to be set for either the center, start or stop

frequency depending on the Frequency Type setting.

Valid Range for the center frequency:

0 to 2.6 GHz for the R3261A, R3261AN, R3361A and R3361AN

0 to 3.6 GHz for the R3261B and R3361B

Valid Range for the start frequency:

-200 MHZ to 2.6 GHz for the R3261A, R3261AN, R3361A and R3361AN

-200 MHz to 3.6 GHz for the R3261B and R3361B

Valid Range for the stop frequency:

0 to 2.8 GHz for the R3261A, R3261AN, R3361A and R3361AN

0 to 3.8 GHz for the R3261B and R3361B

Default Value: 1 GHz

Note : the frequency is always expressed in Hz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Frequency Type) out of range.

BFFC0003  Parameter 3 (Frequency Value) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_FreqCounter

ViStatus AVR3x61_Set_FreqCounter (ViSession instrumentHandle,

ViBoolean counterActivation,

ViInt16 counterResolution);

Purpose

This function is used to activate/deactivate the frequency counter and to

select its resolution.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

counterActivation

Variable Type       ViBoolean

This binary switch is used to activate or deactivate the frequency

counter.

Valid Range :

VI_ON  (1) - ON

VI_OFF (0) - OFF (Default)

counterResolution

Variable Type       ViInt16

This slide is used to select the frequency counter's resolution

Valid Range:

0 - 1 kHz  (Default)

1 - 100 Hz

2 - 10 Hz

3 - 1 Hz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Counter Activation) out of range.

BFFC0003  Parameter 3 (Counter Resolution) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_FreqOffset

ViStatus AVR3x61_Set_FreqOffset (ViSession instrumentHandle,

ViBoolean frequencyOffsetActivation,

ViReal64 frequencyOffsetHz);

Purpose

This function is used to set up the Frequency Offset

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

frequencyOffsetActivation

Variable Type       ViBoolean

This binary switch specifies whether to activate or deactivate the

frequency offset.

Valid Range :

VI_ON  (1): Active

VI_OFF (0): Inactive (Default)

frequencyOffsetHz

Variable Type       ViReal64

Value of the frequency offset.

Valid Range: 0 (Default) to +/-9.999 GHz

Default Value: 0 Hz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Freq. Offset Activation) out of range.

BFFC0003  Parameter 3 (Frequency Offset) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_FreqStepSize

ViStatus AVR3x61_Set_FreqStepSize (ViSession instrumentHandle,

ViBoolean typeofControl,

ViReal64 frequencyStepHz);

Purpose

This function is used to set up the frequency step size.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofControl

Variable Type       ViBoolean

This binary switch lets the user choose whether he determines himself

the value of the frequncy step or he lets the spectrum analyzer

choose this parameter

Data range:

VI_ON  (1) - Auto (Default)

VI_OFF (0) - Manual

Note : In Auto mode, the spectrum analyzer automatically adjusts the

step size to 10% of the center frequency.

frequencyStepHz

Variable Type       ViReal64

Value of the frequency step size (center frequency setting).

Valid Range:

0 to 2.6 GHz for the R3261A, R3261AN, R3361A and R3361AN

0 to 3.6 GHz for the R3261B and R3361B

Default Value : 0 Hz

Note : This control is only valid when "Type of control" is on

"Manual".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Control) out of range.

BFFC0003  Parameter 3 (Frequency Step) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_LinearSpan

ViStatus AVR3x61_Set_LinearSpan (ViSession instrumentHandle,

ViInt16 linearSpanType, ViReal64 spanValueHz);

Purpose

This function lets the user specify the frequency span.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

linearSpanType

Variable Type       ViInt16

The Slide "Type of Control" lets the user choose between : Manual,

Full Span and Zero Span.

Valid Range :

0 - Manual (Default)

1 - Full Span

2 - Zero Span

Notes :

(1) The other parameter of this function is "Span Value" which is

only valid when "0-Manual" is selected by this control.

(2) Full Span : Center Frequency = middle of the frequency range and

Span = frequency range.

(3) Zero Span : Span = 0 Hz, Time Domain analysis.

spanValueHz

Variable Type       ViReal64

This control is used to set up the value of the span when the "Type

of Control" is on "Manual".

Valid Range :

1 kHz to 2.6 GHz for the R3261A, R3261AN, R3361A and R3361AN

1 kHz to 3.6 GHz for the R3261B and R3361B

Default Value: 100 MHz

Note : The frequency span is always expressed in Hz.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Span Type) out of range.

BFFC0003  Parameter 3 (Span Value) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_LogSpan

ViStatus AVR3x61_Set_LogSpan (ViSession instrumentHandle, ViInt16 logStart,

ViReal64 logStop);

Purpose

This function allows the user to work with a logarithmic span.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

logStart

Variable Type       ViInt16

This control is used to specify the frequency Log Start value.

Valid Range:

0 - 10 kHz

1 - 100 kHz

2 - 1 MHz

3 - 10 MHz

4 - 100 MHz

Default Value : 3 - 10 MHz

logStop

Variable Type       ViReal64

This control is used to specify the frequency Log Stop value.

Valid Range: the possible values depends on the log start value

Default Value : 10 MHz

Notes :

(1) if log start = 10 kHz, log stop = 100 kHz, 1 MHz or 10 MHz

(2) if log start = 100 kHz, log stop = 1 MHz, 10 MHz or 100 MHz

(3) if log start = 1 MHz, log stop = 10 MHz, 100 MHz or 1000 MHz

(4) if log start = 10 MHz, log stop = 100 MHz or 1000 MHz

(5) if log start = 100 MHz, log stop = 1000 MHz

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Log Start) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_MarkerDeltaTo

ViStatus AVR3x61_Set_MarkerDeltaTo (ViSession instrumentHandle,

ViInt16 markerDeltaTo);

Purpose

This function is used to set one of the following parameters :

span, CF step or marker step to be the delta marker frequency.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

markerDeltaTo

Variable Type       ViInt16

selects the parameter that will be assigned to the current delta

marker value.

Valid Range :

0 - Span (Default)

1 - CF Step

2 - Marker Step

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Marker Delta To) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_MeasurementWindow

ViStatus AVR3x61_Set_MeasurementWindow (ViSession instrumentHandle,

ViBoolean showHideWindow,

ViReal64 xaxisPosition,

ViReal64 xaxisWidth,

ViReal64 yaxisPosition,

ViReal64 yaxisWidth);

Purpose

This function is used to specify the positions and the widths of the

Measurement Window along the two axis and to activate or deactivate it.

Note : this function can be used only if you are working with a non nul

span.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

showHideWindow

Variable Type       ViBoolean

This binary switch is used to show or hide the Measurement Window.

Valid Range:

VI_ON  (1) - Show

VI_OFF (0) - Hide (Default)

xaxisPosition

Variable Type       ViReal64

Specifies the position of the Measurement Window along the X-axis.

Valid Range : depends on the frequency range studied

Default Value : 0

Notes :

(1) this function can be used only if you are working with a non nul

span.

(2) window position = position of the window's center and unit is Hz.

xaxisWidth

Variable Type       ViReal64

Specifies the width of the Measurement Window.

Valid Range : depends on the frequency range studied

Default Value : 0

Notes :

(1) this function can be used only if you are working with a non nul

span.

(2) window width unit is Hz.

yaxisPosition

Variable Type       ViReal64

Specifies the center's position of the Measurement Window along the

Y-axis.

Valid Range : depends on the reference level and the reference unit.

Default Value : 0

yaxisWidth

Variable Type       ViReal64

Specifies the width of the Measurement Window along the Y-axis.

Valid Range : depends on the reference level and the reference unit.

Default Value : 0

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Show/Hide Window) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_NormalMarker

ViStatus AVR3x61_Set_NormalMarker (ViSession instrumentHandle,

ViReal64 positionHzors);

Purpose

This panel is used to activate and to position the Normal Marker

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

positionHzors

Variable Type       ViReal64

Specifies where to position the Normal Marker : this is either a

frequency (Hz) or a time (s), depending on the current span.

Note : it is IMPOSSIBLE to have a negative position !!!!

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_PeakSearchOptions

ViStatus AVR3x61_Set_PeakSearchOptions (ViSession instrumentHandle,

ViBoolean continuousPeakSearch,

ViBoolean signalTrack,

ViReal64 peakDeltaXValue,

ViReal64 peakDeltaYValue);

Purpose

This function is used to determine the value of Peak delta X and Peak

Delta Y and to activate or deactivate the continuous peak search and the

signal track.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

continuousPeakSearch

Variable Type       ViBoolean

This binary switch allows to execute a continuous search of the peak

or not

Valid Range :

VI_ON  (1) : Active

VI_OFF (0) : Inactive (Default)

signalTrack

Variable Type       ViBoolean

This binary switch allows to use the option Signal Track.

Valid Range :

VI_ON  (1): Active

VI_OFF (0): Inactive (Default)

peakDeltaXValue

Variable Type       ViReal64

This control is used to set up the value of Peak deltaX.

Valid Range: 1 to 700

Default Value : 15

peakDeltaYValue

Variable Type       ViReal64

This control is used to set up the value of Peak deltaY.

Valid Range: 1 to 400

Default Value : 40

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Continuous Peak Search) out of range.

BFFC0003  Parameter 3 (Signal Track) out of range.

BFFC0004  Parameter 4 (Peak Range) out of range.

BFFC0005  Parameter 5 (Peak DeltaY Value) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_RBW

ViStatus AVR3x61_Set_RBW (ViSession instrumentHandle, ViInt16 RBWSetting);

Purpose

This function is used to set up the bandwidth of the filter of

resolution.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

RBWSetting

Variable Type       ViInt16

This control is used to specify the value of the bandwidth of the

filter of resolution (RBW).

Valid Range:

0 - AUTO    (Default)

1 - 30 Hz

2 - 100 Hz

3 - 300 Hz

4 - 1 kHz

5 - 3 kHz

6 - 10 kHz

7 - 30 kHz

8 - 100 kHz

9 - 300 kHz

10 - 1 MHz

Note : Automatic RBW can be selected in which case the analyzer

will automatically choose the RBW that fits best the current SPAN

selection.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (RBW Setting) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_RefLevel

ViStatus AVR3x61_Set_RefLevel (ViSession instrumentHandle,

ViReal64 referenceLeveldBxorV);

Purpose

This function is used to set up the Reference Level value but not the

reference unit.

Note : the reference unit can be chosen by using the function "set

reference unit".

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceLeveldBxorV

Variable Type       ViReal64

Set up the Reference Level.

Data Range : depends on the analyzer used. See the Spectrum Analyzer

User Manual

Default Value : 0 with the current unit.

Note : The unit can be set in the function "Set Reference Unit".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Reference Level) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_RefOffset

ViStatus AVR3x61_Set_RefOffset (ViSession instrumentHandle,

ViBoolean refLevelOffsetActivation,

ViReal64 referenceLevelOffsetdB);

Purpose

This function is used to set the reference level Offset and to activate

or deactivate it.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

refLevelOffsetActivation

Variable Type       ViBoolean

This binary switch allows to activate or deactivate the reference

level offset.

Valid Range :

VI_ON  (1): Active

VI_OFF (0): Inactive (Default)

referenceLevelOffsetdB

Variable Type       ViReal64

Value of the Reference Level Offset.

Valid Range : 0 to  +/- 100 dB

Default Value: 0 dB

Note : The unit is always the dB

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Ref Lev Offset Activation) out of range.

BFFC0003  Parameter 3 (Reference Level Offset) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_RefUnit

ViStatus AVR3x61_Set_RefUnit (ViSession instrumentHandle, ViInt16 referenceUnit);

Purpose

This function is used to set up the reference unit. The unit remains

valid until it changed using this same function.

Notes :

(1) It is thus unnecessary to set up the unit each time the reference

level changes.

(2) This function can be used only if a logarithmic scale is used

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

referenceUnit

Variable Type       ViInt16

Selects the unit of the reference level.

Valid Range :

0 - dBm (Default)

1 - dBmV

2 - dBV

3 - dBVemf

4 - dBpW

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Reference Unit) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_Scale

ViStatus AVR3x61_Set_Scale (ViSession instrumentHandle, ViInt16 scale);

Purpose

This function is used to set up the scale of the Y axis.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

scale

Variable Type       ViInt16

Selects the scale of the Y axis. Either described in dB per screen

division or linearly.

Valid Range:

dB / Div

0 - 10 dB/ (Default)

1 - 5 dB/

2 - 2 dB/

3 - 1 dB/

4 - 0.5 dB/

5 - 0.2 dB/

6 - 0.1 dB/

Linear

7 - Lin x1

8 - Lin x1.6

9 - Lin x4

10 - Lin x8

Notes :

(1) When the scale, previously logarithmic, becomes linear the

reference unit changes to Volts.

(2) When the scale, previously linear, becomes logarithmic the

reference unit changes from Volts to dBm.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Scale) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_SweepMode

ViStatus AVR3x61_Set_SweepMode (ViSession instrumentHandle, ViBoolean sweepMode);

Purpose

This function is used to activate or deactivate the Window Sweep. In

these 2 cases, a continuous sweep is done.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

sweepMode

Variable Type       ViBoolean

This binary switch lets the user choose whether the sweep is

performed only within the Measurement Window or not.

Valid Range:

VI_ON  (1) - Normal sweep (Default)

VI_OFF (0) - Window sweep

Note : Set up the window using the function "Set Measurement Window".

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Sweep Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_SweepTime

ViStatus AVR3x61_Set_SweepTime (ViSession instrumentHandle,

ViBoolean typeofControl, ViReal64 sweepTimesec);

Purpose

This function is used to set up the Sweep Time.

Automatic sweep time selection can be specified, in which case a sweep

time is chosen, by the analyzer, depending on the current setting of RBW,

VBW and SPAN.

SPAN/(RBW * Min(RBW,VBW) * 0.5) = SWEEP TIME. (AUTO mode)

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

typeofControl

Variable Type       ViBoolean

The Type of Control lets the user choose betwen a manual control or

an automatic control of the sweep time.

Valid Range:

VI_OFF (0) - Manual

VI_ON  (1) - AUTO (Default)

Note : Automatic control means that the analyzer itself will

determine the best sweep time depending on the span, RBW and VBW

settings.

sweepTimesec

Variable Type       ViReal64

Value of the sweep time.

Valid Range :

30e-3 to 1000 seconds

Default Value : 50e-3 seconds.

Notes :

(1) This parameter has no importance if AUTO is selected.

(2) The sweep Time is always expressed in second

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Type of Control) out of range.

BFFC0003  Parameter 3 (Sweep Time) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_TraceAMode

ViStatus AVR3x61_Set_TraceAMode (ViSession instrumentHandle, ViInt16 traceAMode,

ViInt16 averageCount, ViBoolean averageMode);

Purpose

This function is used to set up the TRACE A mode. Once set up the mode

becomes active immediately.

Note : if sweep mode is SINGLE, then mode activation will become visible

at the next sweep!

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

traceAMode

Variable Type       ViInt16

Selects the TRACE A mode.

Valid Range :

0 - Write (Default)

1 - View

2 - Blank

3 - Max Hold

4 - Averaging.

averageCount

Variable Type       ViInt16

Sets the number of sweeps over which averaging should be performed.

Data Range : 2 to 1000

Default Value : 20

Note : This setting is ignored if the trace A mode is not on

"Averaging".

averageMode

Variable Type       ViBoolean

Specifies the average mode.

Valid Range :

VI_OFF (0) - Single

VI_ON  (1) - Continuous (Default)

Note : This setting is ignored when trace A mode is not on

"Averaging".

Let the following notation be defined:

n      current averaging count

N      Averaging count specified (see control Averaging Count)

Yn     Trace data for nth average

<Yn>   Averaged data for nth average

<Yn-1> Averaged data for n-1 th average

Sigma  Sum of all the data up to the nth point.

Algorithm 1

[n<=N]  <Yn> = Sigma/n

Algorithm 2

[N<n]   <Yn> = ((N-1) - <Yn-1>) / N + <Yn> / N

* When Continuous is selected averaging is repeated using algorithm 2

even after the specified average count has been reached.

* When Single is selected the analyzer will automatically leave the

averaging mode for View mode, as soon as the specified average count

has been reached.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace A Mode) out of range.

BFFC0003  Parameter 3 (Average Count) out of range.

BFFC0004  Parameter 4 (Average Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_TraceBMode

ViStatus AVR3x61_Set_TraceBMode (ViSession instrumentHandle, ViInt16 traceBMode,

ViInt16 averageCount, ViBoolean averageMode);

Purpose

This function is used to set up the TRACE A mode. Once set up the mode

becomes active immediately.

Note : if sweep mode is SINGLE, then mode activation will become visible

at the next sweep!

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

traceBMode

Variable Type       ViInt16

Selects the TRACE B mode.

Valid Range :

0 - Write (Default)

1 - View

2 - Blank

3 - Max Hold

4 - Averaging.

averageCount

Variable Type       ViInt16

Sets the number of sweeps over which averaging should be performed.

Data Range : 2 to 1000

Default Value : 20

Note : This setting is ignored if the trace B mode is not on

"Averaging".

averageMode

Variable Type       ViBoolean

Specifies the average mode.

Valid Range :

VI_OFF (0) - Single

VI_ON  (1) - Continuous (Default)

Note : This setting is ignored when trace B mode is not on

"Averaging".

Let the following notation be defined:

n      current averaging count

N      Averaging count specified (see control Averaging Count)

Yn     Trace data for nth average

<Yn>   Averaged data for nth average

<Yn-1> Averaged data for n-1 th average

Sigma  Sum of all the data up to the nth point.

Algorithm 1

[n<=N]  <Yn> = Sigma/n

Algorithm 2

[N<n]   <Yn> = ((N-1) - <Yn-1>) / N + <Yn> / N

* When Continuous is selected averaging is repeated using algorithm 2

even after the specified average count has been reached.

* When Single is selected the analyzer will automatically leave the

averaging mode for View mode, as soon as the specified average count

has been reached.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trace B Mode) out of range.

BFFC0003  Parameter 3 (Average Count) out of range.

BFFC0004  Parameter 4 (Average Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_TraceDetector

ViStatus AVR3x61_Set_TraceDetector (ViSession instrumentHandle,

ViInt16 detectorMode);

Purpose

This function is used to select the Trace Detector Mode that will be used

to display the data.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

detectorMode

Variable Type       ViInt16

This control determines the detector mode that will now be used.

Valid Range:

0 - Normal (Default)

1 - Positive

2 - Negative

3 - Sample

Notes:

(1) Normal or Posi-Nega:the level assigned to the screen point is

alternately the maximum or the minimum level measured since the last

point.

(2) Positive:the maximum level measured since the last point is

assigned to the screen point

(3) Negative:the minimum level measured since the last point is

assigned to the screen point

(4) Sample:the level at the screen point is taken for the screen

point.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Detector Mode) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_Trigger

ViStatus AVR3x61_Set_Trigger (ViSession instrumentHandle, ViInt16 triggerSource);

Purpose

This function is used to choose the trigger between free run, line,

video, TV_V, eternal and single

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

triggerSource

Variable Type       ViInt16

Selects the type of trigger

Data Range:

0 - Free run (Default)

1 - Line

2 - Video

3 - TV_V

4 - External

5 - Single

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (Trigger Source) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_Set_VBW

ViStatus AVR3x61_Set_VBW (ViSession instrumentHandle, ViInt16 VBWSetting);

Purpose

This function is used to set up the bandwidth of the video filter.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

VBWSetting

Variable Type       ViInt16

This control is used to specify the bandwidth video filter value

(VBW).

Valid Range

0 - AUTO  (Default)

1 - 1 Hz

2 - 10 Hz

3 - 100 Hz

4 - 1 kHz

5 - 10 kHz

6 - 100 kHz

7 - 1 MHz

Notes :

(1) Automatic VBW can be selected in which case the analyzer will

automatically choose VBW according to RBW.

(2) For noise averaging, VBW is often chosen 1/10 under RBW (RBW =

100 kHz -> VBW = 10 kHz)

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFC0002  Parameter 2 (VBW Setting) out of range.

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.

________________________________________________________________________________

AVR3x61_WriteInstrData

ViStatus AVR3x61_WriteInstrData (ViSession instrumentHandle,

ViString writeBuffer);

Purpose

This function writes commands and queries to the instrument to modify

parameters and query device settings.

Parameter List

instrumentHandle

Variable Type       ViSession

This control accepts the Instrument Handle returned by the Initialize

function to select the desired instrument driver session.

Default Value:  None

writeBuffer

Variable Type       ViString

The user can use this control to send common commands and queries to

the instrument. This control can also be used to write any valid

command to the instrument.

Default Value:  "IP"

Note : The command or query to be sent to the instrument may be a

literal enclosed in double quotes i.e. "IP" or may be contained in a

variable of type string.

Return Value

This control contains the status code returned by the function call.

Status Codes:

Status    Description

-------------------------------------------------

0  No error (the call was successful).

BFFF0000  Miscellaneous or system error occurred.

BFFF000E  Invalid session handle.

BFFF0015  Timeout occurred before operation could complete.

BFFF0034  Violation of raw write protocol occurred.

BFFF0035  Violation of raw read protocol occurred.

BFFF0036  Device reported an output protocol error.

BFFF0037  Device reported an input protocol error.

BFFF0038  Bus error occurred during transfer.

BFFF003A  Invalid setup (attributes are not consistent).

BFFF005F  No listeners condition was detected.

BFFF0060  This interface is not the controller in charge.

BFFF0067  Operation is not supported on this session.