Power Monitor Sample Software for the Advantest U3751, U3771 and U3772 Spectrum Analyzers

Advantest Corporation

General Outline:

This software does Power measurement by controlling the output of a SG and displaying measured result as a graph showing measurement result, measurement time, and linearity. Deselecting the SG check box disables the SG control leaving the output power fixed. This allows for measurement of variation over time.

GPIB address: SPA=No.8　SG=No.6　For this sample software the addresses are fixed. (You can change in the Source File)

１　　　  ２　　　　  ３　　　 ４　　　  ５　　　　      ６　　　  ７　　	８

How to use the Power Monitor Panel

１．Zero Span:　 Places the spectrum analyzer in Zero Span mode.  The measurement time is shorter than using the normal Channel Power mode. Measurement values are for relative not absolute values. This is a measure of linearity not overall level accuracy.  At the default position, it measures the power while changing the output level of the SG from +4dBm to –56dBm in 4dB steps. Measurement is made using the Average Power Measurement function (AVG)

Note: The maximum output power of a W-CDMA MS (mobile Station) is +24dBm.Many radios and other applications require far higher power than 4dBm. However, for most standard SG’s, an output +24dBm would be impossible, so for the purposes of this demonstration software the starting point is +4dBm.

２．CH Power:　 Measurements are made using the built in Channel Power Mode. This is slower than the zero span mode but should improve the overall accuracy.

３．Carrier.:     Set Carrier Frequency: This is the center frequency of the signal to be measured.

４．Input Lvl.:   Set Input level: This is the beginning level of the SG output. If the “SG access” box is checked the program will start at his level and then decrease the SG level by 4dB until the output reaches -56dBm. If the SG access box is unchecked the level of the SG will remain at the valued entered.

５．Ext. ATT:	External Attenuator value: If an external attenuator is in line with the instrument, entering the value here will offset the value measured. The input level (4) is the output of the generator. Compensating there will cause an inaccurate level scale.

６．SG Access:   Signal Generator Access : SG is controlled from the program if this box is checked. If left unchecked the SG level will remain fixed.

７．Level Offset:  Offset loss: This allows the entry of a loss value for the difference between the output of the SG and the power actually delivered to the spectrum analyzer.

８．Average：    Averaging: The measurement will show the results of 10 averages.